भारतीय मानक

Indian Standard

IS 3665 : 2024

इनवॉल्यूट साइडेड स्प्लाइन्स — आयाम

( पहला पुनरीक्षण )

Involute Sided Splines — Dimensions

( First Revision )

ICS 21.120.10

BIS 2024

भारतीय मानक ब्यूरो

BUREAU OF INDIAN STANDARDS

मानक भवन, 9 बहादुर शाह ज़फर मार्ग, नई दिल्ली ‒ 110002

MANAK BHAVAN, 9 BAHADUR SHAH ZAFAR MARG

NEW DELHI ‒ 110002

www.bis.gov.in     www.standardsbis.in

March 2024                                		     Price Group X

Transmission Devices Sectional Committee, PGD 33

FOREWORD

This Indian Standard (First Revision) was adopted by the Bureau of Indian Standards after the draft finalized by the Transmission Devices Sectional Committee had been approved by the Production and General Engineering Division Council.

This standard was first published in 1966. This revision has been taken up to keep pace with the latest technological developments and international practices.

Splined shafts generally have the following three types of applications:

- Coupling shafts when relatively heavy torques are to be transmitted without slippage;

- Transmitting power to floating or permanently fixed gears, pulleys and other rotating members; and

- Coupling parts that may require frequent removal for indexing or change of angular position.

External and internal splines are extensively used in the automobile, machine tools and other industries. This standard has been formulated to rationalize the production and to facilitate interchangeability of external and internal splines.

This standard deals with involute sided splines of 30° pressure angle for general engineering purposes. Separate standards on straight sided splines have already been formulated (see IS 2327 : 1993 and IS 2610 : 1989).

The dimensions and fits given in the tables are based on the basic hole system. In this system the dimensions of the internal splines are the basis and variations in fit are obtained by varying the allowance on the external splines.

The tolerances for the spline tooth thickness and space width are given in Table 12. The instructions for the use of table is explained in Annex A. The tolerance on major and minor diameters shall be according to the system of limits and fits specified in IS 919 (Part 1) : 2014 and IS 919 (Part 2) : 2014.

Separate standards on gauging practice and the relevant manufacturing tools for splines are under preparation.

As far as possible, the major diameters of external splines are given, so as to end in numbers 0, 2, 5 and 8 which are standard diameters for ball bearings.

The composition of Committee responsible for formulation of this standard is given in Annex B.

For the purpose of deciding whether a particular requirement of this standard is complied with the final value, observed or calculated, expressing the result of a test or analysis shall be rounded off in accordance with                                   IS 2 : 2022 ‘Rules for rounding off numerical values (second revision).’

Indian Standard

INVOLUTE SIDED SPLINES — DIMENSIONS

( First Revision )

1 SCOPE

1.1 This standard specifies the dimensions, for straight involute spline of 30˚ pressure angle, with three different types of fits,namely,major diameter fit, minor diameter fit and side fit.

1.2 Involute splines of modules (l), 1.25, (l.5), 2, (2.5), 3, (4), 5, (6), 8 and (10) are covered in this standard. The values given within brackets are non‒preferred.

2 REFERENCES

The following standards given below contain provisions, which through reference in this text, constitute provisions of this standard. At the time of publication, the editions indicated were valid. All standards are subject to revision and encouraged to investigate the possibility of applying the most recent edition of these standards:

| IS No. | Title |
| --- | --- |
| IS 919 | Geometrical product specifications (GPS) — ISO code system for tolerances on linear sizes |
| (Part 1) : 2014 | Basis of tolerance, deviation and fits (third revision) |
| (Part 2) : 2014 | Tables of standard tolerance classes and limit deviation for holes and shafts (second revision) |
| IS 2327 : 1993 | Straight — Sided splines for cylindrical shafts with internal centering — Dimensions, tolerances and verification (first revision) |
| IS 2610 : 1989 | Power transmission — Straight sided splines for machine tools — Dimensions (first revision) |

3 DEFINITIONS

For the purpose of this standard, following definitions shall apply:

3.1 Actual Space Width — The circular width on the pitch circle of any single space (see Fig. 1).

3.2 Actual Tooth Thickness — The circular thickness on the pitch circle of any single tooth (see Fig. 2).

a) Each space is basic width                                        b) Perfect external spline with basic

tooth thickness interferes at X

c) The perfect external spline fits in any position, if all spaces of the internal spline are widened by the amount of interference.

Fig. 1 Efffct of Internal Splinb Errors

Fig. 2 Effect of External Spline Errors

3.3 Base Circle Diameter — The diameter of the circle from which involute spline tooth profiles are constructed.

3.4 Effective Clearance — The difference between the effective space width of the internal spline and the effective tooth thickness of the mating external spline (see Fig.3).

3.5 Effective Error — The accumulated effect of the spline errors on the fit with the mating parts.

3.6 Effective Space Width of an Internal Spline — The circular tooth thickness on the pitch circle of an imaginary perfect external spline which would fit the internal spline without looseness or interference                             (see Fig. 1).

3.7 Effective Tooth Thickness of an External Spline — The circular space width on the pitch circle of an imaginary perfect internal spline which would fit the external spline without looseness or interference (see Fig.2).

3.8 Error Allowance — The permissible effective error.

3.9 Involute Spline — The spline having teeth with involute profiles.

3.10 Machining Tolerance — The permissible variation in actual space width or actual tooth thickness                                     (see Fig. 3).

3.11 Main Pressure Angle (a0) — The pressure angle at the pitch point.

3.12 Major Diameter — The diameter of the outermost surface of the spline. It is the root diameter of the internal spline or the tip diameter of the external spline.

Fig. 3 Fit Diagram of Spline Assembly

120 × 114 × 38 × 8HE/8eb IS 3665

3.13 Minor Diameter — The diameter of the innermost surface of the spline. It is the tip diameter of the internal spline or the root diameter of the external spline.

3.14 Module (m) — The ratio of the pitch circle diameter to the number of spline teeth.

3.15 Nominal Clearance — The difference between the actual space width of an internal spline and the actual tooth thickness of the mating external spline. This does not define the fit between the mating members, because of the effect of errors.

3.16 Pitch Circle Diameter — The diameter of an imaginary reference circle (pitch circle) from which all transverse dimensions are derived.

3.17 Pitch Point — The point of intersection of the spline tooth profile with the pitch circle.

3.18 Pressure Angle — The acute angle between a line tangent to a profile of the spline and a radial line through the point of tangency. Unless otherwise specified the pressure angle shall mean the main pressure angle.

3.19 Profile Displacement (xm) — The displacement of the basic rack either away or towards the reference cylinder and is denoted by xm. The former is taken as positive and latter as negative profile displacement.

3.20 Spline — A machine element consisting of integral keys (spline teeth) or keyways (spaces) equally spaced around a circle or a portion thereof.

3.21 Total Tolerance — The sum of the machining tolerance and the error allowance (see Fig. 3).

4 TYPES

4.1 External Spline

An involute spline whose tip surface is external to the root surface.

4.2 Internal Spline

An involute spline whose tip surface is internal to the root surface.

5 EFFECTIVE AND ACTUAL DIMENSIONS

The effective dimensions and actual dimensions together determine the fit of a spline assembly.

6 DIMENSIONS, TOLERANCES AND FITS

6.1 Reference profile for the different pitches shall be as given in Fig. 4.

6.2 Major Diameter of Internal Spline (d1) — This is the reference diameter of the profile, and the value shall be according to Tables 1 to 11.

6.3 Profile Displacement (xm) — The value of the profile displacement shall be calculated from the following formula (see Tables 1 to 11):

xm = l/2 (d1 ‒‒ m.z ‒‒  l.1 m)

The value shall be from ‒‒  0.05 m to + 0.45 m

6.4 Number of Teeth (z) — The number of teeth shall be calculated from the following formula                                         (see Tables 1 to 11):

z = 1m(d1 ‒‒  2 xm ‒‒  1.1 m)

where

d1 = major diameter of the internal spline.

6.5 Minor Diameter of the Internal Spline (d2) — The value of the minor diameter of the internal spline shall be calculated from the following formula (see Tables 1 to 11):

D2 = m.z + 2 xm ‒‒  0.9 m

= d1 ‒‒ 2 m

Fig. 4 Reference Profile

6.6 Major Diameter of the External Spline (d3) — The value of the major diameter of the external spline shall be calculated from the following formula (see Tables 1 to 11):

d3 = m.z + 2xm + 0.9 m

= d1 ‒‒  0.2 m

6.7 Minor Diameter of the External Spline s (d4) — Minor diameter of the external splines shall be calculated from the following formula (see Tables 1 to 11):

d4 = m.z + 2 xm ‒‒ 1.1 m

= d1  ‒‒  2.2 m

6.8 Space Width and Tooth Thickness (lo and So) — The value of the tooth thickness and space width shall be calculated from the following formula (see Tables 1 to 11):

l0 and s0 = mπ2 + 2. xm. tan ao

6.9 The measuring pin diameter, the measurement over pins for the external splines and the measurement between pins for the internal splines shall be as given in Tables 1 to 11 (see Fig. 5).

6.9.1 Tolerance on Measurement Over Pins — The tolerance values on effective and actual measurement over pins shall be obtained by multiplying the tolerance values on the effective and actual tooth thickness by the deviation factor, f (Tables 1 to 11).

6.9.2 Tolerance on Measurement Between Pins — The tolerance on effective and actual measurement between pins shall be obtained by multiplying the tolerance values on the effective and actual space width by the deviation factor (Tables 1 to 11).

Internal Spline                                                        External Spline

Fig. 5 Measurement Between Pins and Measurement Over Pins

6.10 The thickness of external splines over a specified number of teeth shall be according to Tables 1 to 11.

The tolerance on the effective and actual thickness of external splines over a specified number of teeth shall be the product of the tolerance on the effective and actual tooth thickness and the deviation factor 0.866.

6.11 Types of Fits

6.11.1 Major Diameter Fit — In this type of fit, the major diameter of the external and internal splines has the same nominal diameter d1. There shall be circular clearance between the internal spline space width and external spline tooth thickness.

6.11.1.1 The recommended tolerance values for the internal spline minor diameters (d2) shall be Hl1 and the tolerance values for the external spline minor diameters (d4) shall be h14. An example of major diameter fit is given in Fig. 6.

Fig.6 Example of Major Diameter Fit

6.11.2 Minor Diameter Fit — In this type of fit, the minor diameter of the internal and external splines shall have the same nominal diameter d2. There shall be circular clearance between the internal spline space width and external spline tooth thickness.

6.11.2.1 The recommended tolerance values for the internal spline major diameters (d1) shall be H14 and the tolerance values for the external spline major diameters (d3) shall be hl1. An example of minor diameter fit is given in Fig. 7.

6.11.3 Side Fit — In this type of fit, the mating members contact on the sides of the teeth only. There shall be clearance between the major diameters and minor diameters. The tolerance values for spindle tooth thickness and space width for the spline qualities 7, 8, 9 and 10 shall be as given in Table 12.

NOTE — Instructions for the use of Table 12 is given in Annex A.

Fig. 7 Example of Minor Diameter Fit

6.11.3.1 The recommended tolerance values for the major diameters of internal spline (dl) shall be h14, the minor diameters of internal spline (d2) shall be h11, the major diameters of external spline (d3) shall be h11 and the minor diameters of external spline (d4) shall be h14. An example of side fit is shown in Fig. 8.

Fig. 8 Example of Side Fit

6.11.4 A typical example of effective tooth thickness, effective space width, actual tooth thickness and the actual space width for three types of fits, namely, press fit, locating fit and sliding fit, is shown in Table 13.

7 DESIGNATION

7.1 Side Fit

7.1.1 An involute sided spline of a spline assembly of side fit shall be designated by the type of spline, nominal size (d1 x d2), number of teeth of the spline, the tolerance on the effective and actual dimensions; of space width for internal splines, and of tooth thickness for external spline, followed by the number of this standard.

Example: An external involute spline of nominal size 120 × 114 mm with 38 spline teeth, 	and with the tolerance symbols 8e and 8b on the effective and actual tooth thickness 	shall be designated as:

External Involute Spline 120 × 114 × 38 × 8eb IS 3665

7.1.2 A side fitted spline assembly shall be designated by the nominal size, number of teeth, d, the spline, the effective and actual fit of the spline teeth and the number of this standard.

Example:

A spline assembly of nominal size 120 × 114, with 38 spline teeth and the fit 	8 HE/8eb on the effective and actual spline teeth shall be designated as:

Spline Assembly 120 × 114 × 38 × 8 HE/8eb IS: 3665

7.2 Diameter Fit

Spline assembly of major and minor diameter fit shall be designated as in 6.1.2 along with the value of fit on their respective diameters.

Examples:

- A spline assembly of major diameter fit of H7/h6 of external involute spline, 120 × 114 × 38 × 8eb and internal involute spline 120 × 114 × 38 × 8 HE shall be designated as:

Spline Assembly 120 H7/h6 × 114 × 38 × 8 HE/8eb IS: 3665

- A spline assembly of minor diameter fit of H7/h6 of external involute spline, 120 × 114 × 38 × 8eb and internal involute spline 120 × 114 × 38 × 8 HE shall be designated as:

Spline Assembly 120 × 114 H7/h6 × 38 × 8 HE/8eb IS: 3665

8 BIS CERTIFICATION MARKING

The product(s) conforming to the requirements of this standard may be certified as per the conformity assessment schemes under the provisions of the Bureau of Indian Standards Act, 2016 and the Rules and Regulations framed thereunder, and the product(s) may be marked with the Standard Mark.

Table 1 Dimensions for Involute Splines of Module 1

(Clauses 6.2 to 6.10)

All dimensions in millimeters.

| Sl No. | Nominal Size |  |  |  |  |  |  |  |  |  | Internal Spline |  |  | External Spline |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Pin Dia | Measur‒ement Between pine | Deviation Factor | Pin Dia | Measur‒ement Over Pins | Deviation Factor | Tooth Thickness Over z' Teeth |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |
|  | d1 × d2 | z | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so | d | Mi | fi | d | Ma | fa | Z' |  |
|  | 8 × 6 10 × 8 | 6 8 | 6 8 | 5.196 6.928 | 7.8 9.8 | 5.8 7.8 | 7.86 9.86 | 5.94 8.94 | + 0.45 + 0.45 | 2.090 2.090 | 1.75 1.75 | 4.367 6.368 | 1.64 1.66 | 4.00 3.00 | 14.173 14.103 | 1.01 1.11 | 2 2 | 4.810 4.903 |
|  | 12 × 10 15 × 13 | 10 13 | 10 13 | 8.660 11.258 | 11.8 14.8 | 9.8 12.8 | 11.86 14.87 | `9.94 12.93 | + 0.45 + 0.45 | 2.090 2.090 | 1.75 1.75 | 8.369 11.273 | 1.67 1.67 | 2.75 2.50 | 15.615 17.992 | 1.17 1.24 | ‒‒ 3 | ‒‒ 7.857 |
|  | 17 × 15 18 × 16 | 15 16 | 15 16 | 12.990 13.856 | 16.8 17.8 | 14.8 15.8 | 16.87 17.87 | 14.93 15.93 | + 0.45 + 0.45 | 2.090 2.090 | 1.75 1.75 | 13.286 14.369 | 1.68 1.69 | 2.25 2.25 | 19.431 20.541 | 1.31 1.33 | ‒‒ ‒‒ | ‒‒ ‒‒ |
|  | 20 × 18 22 × 20 | 18 20 | 18 20 | 15.588 17.321 | 19.8 21.8 | 17.8 19.8 | 19.87 21.87 | 17.93 19.93 | + 0.45 + 0.45 | 2.090 2.090 | 1.75 1.75 | 16.370 18.370 | 1.70 1.70 | 2.25 2.25 | 22.568 24.592 | 1.35 1.37 | 4 4 | 10.810 10.903 |
|  | 25 × 25 28 × 26 | 24 26 | 24 26 | 20.785 22.517 | 24.8 27.8 | 22.8 25.8 | 24.87 27.88 | 22.93 25.92 | ‒ 0.05 + 0.45 | 1.513 2.090 | 1.75 1.75 | 21.311 24.370 | 2.00 1.71 | 2.00 2.00 | 27.109 29.982 | 1.54 1.46 | ‒‒ 5 | ‒‒ 13.903 |
|  | 30 × 28 32 × 30 | 28 30 | 28 30 | 24.249 25.981 | 29.8 31.8 | 27.8 29.8 | 29.88 31.88 | 27.92 29.92 | + 0.45 + 0.45 | 2.090 2.090 | 1.75 1.75 | 26.370 28.370 | 1.71 1.71 | 2.00 2.00 | 31.992 34.002 | 1.47 1.48 | ‒‒ 6 | ‒‒ 16.810 |
|  | 35 × 33 37 × 35 | 34 36 | 34 36 | 29.445 31.177 | 34.8 36.8 | 32.8 34.8 | 34.88 36.88 | 32.92 34.92 | ‒ 0.05 ‒ 0.05 | 1.513 1.513 | 1.75 1.75 | 31.331 33.334 | 1.90 1.89 | 2.00 2.00 | 37.127 39.129 | 1.59 1.59 | 6 ‒‒ | 16.497 ‒‒ |
|  | 38 × 36 40 × 38 | 36 38 | 36 38 | 31.177 32.909 | 37.8 39.8 | 35.8 37.8 | 37.88 39.88 | 33.92 37.92 | +0.45 +0.45 | 2.090 2.090 | 1.75 1.75 | 34.370 36.370 | 1.72 1.72 | 2.00 2.00 | 40.025 42.032 | 1.51 1.52 | 7 7 | 19.810 19.903 |
|  | 42 × 40 | 40 | 40 | 34.641 | 41.8 | 39.8 | 41.88 | 39.92 | +0.45 | 2.090 | 1.75 | 38.370 | 1.72 | 2.00 | 44.038 | 1.53 | ‒‒ | ‒‒ |

| Sl No. |  | Nominal size |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  | Measurement Between Pine |  | Deviation Factor |  | Pin Dia |  | Measurement Over Pins |  | Deviation Factor |  | Tooth Thickness Over Z' Teeth |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |
|  |  | d1× d2 | z | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so | d |  | Mi |  | fi |  | d |  | Ma |  | fa |  | Z' |  |  |  |
|  |  | 10 × 7.5 12 × 9.5 | 6 8 | 7.50 10.00 | 6.495 8.660 | 9.75 11.75 | 7.25 9.25 | 9.81 11.81 | 7.44 9.44 | + 0.5625 + 0.3125 | 2.613 2.324 | 2.25 2.25 |  | 5.276 7.230 |  | 1.71 2.13 |  | 4.50 3.00 |  | 16.629 15.554 |  | 1.03 1.20 |  | 2 2 |  | 6.013 5.879 |  |
|  |  | 15 × 12.5 17 × 14.5 | 10 12 | 12.50 15.00 | 10.825 12.990 | 14.75 16.75 | 12.25 14.25 | 14.82 16.82 | 12.43 14.43 | + 0.5625 + 0.3125 | 2.613 2.325 | 2.25 2.25 |  | 10.276 12.249 |  | 1.72 1.95 |  | 3.25 2.75 |  | 19.075 20.059 |  | 1.19 1.31 |  | ‒‒ 3 |  | ‒‒ 9.513 |  |
|  |  | 18 × 15.5 20 × 17.5 | 13 14 | 16.25 17.50 | 14.073 15.155 | 17.75 19.75 | 15.25 17.25 | 17.82 19.82 | 15.43 17.43 | + 0.1875 + 0.5625 | 2.180 2.613 | 2.25 2.25 |  | 13.099 15.276 |  | 2.08 1.72 |  | 2.50 3.00 |  | 20.360 23.602 |  | 1.38 1.28 |  | 3 3 |  | 9.446 9.876 |  |
|  |  | 22 × 19.5 25 × 22.5 | 16 18 | 20.00 22.50 | 17.321 19.486 | 21.75 24.75 | 19.25 22.25 | 21.82 24.82 | 19.43 22.43 | + 0.3125 + 0.5625 | 2.324 2.613 | 2.25 2.25 |  | 17.257 20.276 |  | 1.88 1.73 |  | 2.5. 2.75 |  | 24.477 28.050 |  | 1.41 1.36 |  | ‒‒ 4 |  | ‒‒ 13.513 |  |
|  |  | 28 × 25.5 30 × 27.5 | 21 22 | 26.25 27.50 | 22.733 23.816 | 27.75 29.75 | 25.25 27.25 | 27.83 29.83 | 24.42 27.42 | + 0.1875 + 0.5625 | 2.180 2.613 | 2.25 2.25 |  | 23.170 25.276 |  | 1.91 1.73 |  | 2.50 2.75 |  | 30.481 33.103 |  | 1.47 1.40 |  | 4 ‒‒ |  | 13.313 ‒‒ |  |
|  |  | 32 × 29.5 35 × 32.5 | 24 26 | 30.00 32.50 | 25.981 28.146 | 31.75 34.75 | 29.25 32.25 | 31.83 34.83 | 29.42 32.42 | + 0.3125 + 0.5625 | 2.324 2.613 | 2.25 2.25 |  | 27.264 30.276 |  | 1.83 1.73 |  | 2.50 2.50 |  | 34.540 37.477 |  | 1.48 1.46 |  | 5 5 |  | 17.013 17.379 |  |
|  |  | 37 × 34.5 38 × 35.5 | 28 29 | 35.00 36.25 | 30.311 31.393 | 36.75 37.75 | 34.25 35.25 | 36.83 37.83 | 34.42 35.42 | + 0.3125 + 0.1875 | 2.324 2.180 | 2.25 2.25 |  | 32.266 33.200 |  | 1.81 1.85 |  | 2.50 2.50 |  | 39.561 40.540 |  | 1.51 1.53 |  | ‒‒ ‒‒ |  | ‒‒ ‒‒ |  |
|  |  | 40 × 37.5 42 × 39.5 | 30 32 | 37.50 40.00 | 32.476 34.641 | 39.75 41.75 | 37.25 39.25 | 39.83 41.83 | 37.42 39.42 | + 0.5625 + 0.3125 | 2.613 2.324 | 2.25 2.25 |  | 35.276 37.267 |  | 1.73 1.80 |  | 2.50 2.50 |  | 42.502 44.577 |  | 1.48 1053 |  | 6 6 |  | 21.013 20.879 |  |
|  |  | 45 × 42.5 47 × 44.5 | 34 36 | 42.50 45.00 | 36.806 38.971 | 44.75 46.75 | 42.25 44.25 | 44.83 46.83 | 42.42 44.42 | + 0.5625 + 0.3125 | 2.613 2.324 | 2.25 2.25 |  | 40.276 42.268 |  | 1.73 1.79 |  | 2.50 2.50 |  | 47.523 49.591 |  | 1.50 1.54 |  | 7 7 |  | 24.647 24.513 |  |
|  |  | 48 × 45.5 50 × 47.5 | 37 38 | 46.25 47.50 | 40.054 41.136 | 47.75 49.75 | 45.25 47.25 | 47.83 49.83 | 45.42 47.42 | + 0.1875 + 0.5625 | 2.180 2.613 | 2.25 2.25 |  | 43.216 45.276 |  | 1.82 1.73 |  | 2.50 2.50 |  | 50.576 52.540 |  | 1.56 1.52 |  | 7 7 |  | 24.446 24.876 |  |
|  |  | 52 × 49.5 | 40 | 50.00 | 43.301 | 51.75 | 49.25 | 51.83 | 49.42 | + 0.3125 | 2.324 | 2.25 |  | 47.269 |  | 1.79 |  | 2.50 |  | 54.602 |  | 1.56 |  | ‒ |  | ‒ |  |

| Table 3 Dimensions for Involute Splines of Module (1.5) (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  | Measurement Between Pine |  | Deviation Factor |  | Pin Dia |  | Measurement Over Pins |  | Deviation Factor |  | Tooth Thickness Over Z' Teeth |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |  |
|  |  | d1× d2 |  | z |  | d0 |  | db |  | d3 |  | d4 |  | d5 Min |  | d6 Max |  | xm |  | lo = so |  | d |  | Mi |  | fi |  | d |  | Ma |  | fa |  | Z' |  |  |  |
|  |  | 12 × 9 15 × 12 |  | 6 8 |  | 9.0 12.0 |  | 7.794 10.392 |  | 11.7 14.7 |  | 8.7 11.7 |  | 11.76 14.76 |  | 8.94 11.94 |  | + 0.675 + 0.675 |  | 3.136 3.136 |  | 2.75 2.75 |  | 6.180 9.180 |  | 1.77 1.76 |  | 5.50 4.50 |  | 2.173 21.155 |  | 1.02 1.11 |  | 2 2 |  | 7.216 7.355 |  |
|  |  | 17 × 14 18 × 15 |  | 10 10 |  | 15.0 15.0 |  | 12.990 12.990 |  | 16.7 17.7 |  | 13.7 14.7 |  | 16.77 17.77 |  | 13.93 14.93 |  | + 0.175 + 0.675 |  | 2.558 3.136 |  | 2.75 2.75 |  | 10.985 12.181 |  | 2.67 1.76 |  | 3.25 4.00 |  | 2.0593 23.127 |  | 1.31 1.18 |  | 2 ‒ |  | 6.995 ‒ |  |
|  |  | 20 × 17 22 × 19 |  | 12 13 |  | 18.0 19.5 |  | 15.588 16.887 |  | 19.7 21.7 |  | 16.7 18.7 |  | 19.77 21.77 |  | 16.93 18.93 |  | + 0.175 + 0.425 |  | 2.558 2.847 |  | 2.75 2.75 |  | 14.038 16.014 |  | 2.33 1.91 |  | 3.00 3.25 |  | 22.995 25.382 |  | 1.38 1.32 |  | 3 3 |  | 11.216 11.535 |  |
|  |  | 25 × 22 28 × 25 |  | 15 17 |  | 22.5 25.5 |  | 19.486 22.084 |  | 24.7 27.7 |  | 21.7 24.7 |  | 24.77 27. 78 |  | 21.93 24.92 |  | + 0.425 + 0.425 |  | 2.847 2.847 |  | 2.75 2.75 |  | 19.036 22.053 |  | 1.88 1.86 |  | 3.25 3.25 |  | 28.446 31.498 |  | 1.35 1.37 |  | 3 4 |  | 11.675 15.896 |  |
|  |  | 30 × 27 32 × 29 |  | 18 20 |  | 27.0 30.0 |  | 23.383 25.81 |  | 29.7 31.7 |  | 26.7 28.7 |  | 29.78 31.78 |  | 26.92 28.92 |  | + 0.675 + 0.175 |  | 3.136 2.558 |  | 2.75 2.75 |  | 24.181 26.111 |  | 1.75 1.99 |  | 3.25 3.00 |  | 33.532 35.078 |  | 1.36 1.48 |  | 4 4 |  | 16.216 15.855 |  |
|  |  | 35 × 32 37 × 34 |  | 22 23 |  | 33.0 34.5 |  | 28.579 29.878 |  | 34.7 36.7 |  | 31.7 33.7 |  | 34.78 36.78 |  | 31.92 33.92 |  | + 0.175 + 0.425 |  | 2.558 2.847 |  | 2.75 2.75 |  | 29.118 31.087 |  | 1.96 1.83 |  | 3.00 3.00 |  | 38.092 39.939 |  | 1.49 1.46 |  | 4 5 |  | 15.995 20.396 |  |
|  |  | 38 × 35 40 × 37 |  | 24 25 |  | 36.0 37.5 |  | 31.177 32.476 |  | 37.7 39.7 |  | 34.7 36.7 |  | 37.78 39.78 |  | 34.92 36.92 |  | + 0.175 + 0.425 |  | 2.558 2.847 |  | 2.75 2.75 |  | 32.125 34.094 |  | 1.93 1.82 |  | 3.00 3.00 |  | 41.103 42.961 |  | 1.51 1.48 |  | 5 5 |  | 20.216 20.535 |  |
|  |  | 42 × 39 45 × 42 |  | 26 28 |  | 39.0 42.0 |  | 33.775 36.373 |  | 41.7 44.7 |  | 38.7 41.7 |  | 41.78 44.78 |  | 38.92 41.92 |  | + 0.675 + 0.675 |  | 3.136 3.136 |  | 2.75 2.75 |  | 36.181 39.181 |  | 1.74 1.74 |  | 3.00 3.00 |  | 44.972 47.988 |  | 1.46 1.47 |  | 5 6 |  | 20.855 25.076 |  |
|  |  | 47 × 44 48 × 45 |  | 30 30 |  | 45.0 45.0 |  | 38.971 38.971 |  | 46.7 47.7 |  | 43.7 44.7 |  | 46.78 47.78 |  | 43.92 44.92 |  | + 0.175 + 0.675 |  | 20558 30136 |  | 2.75 2.75 |  | 41.138 42.181 |  | 1.88 1.74 |  | 3.00 3.00 |  | 50.130 51.003 |  | 1.54 1.48 |  | 6 6 |  | 24.716 25.216 |  |
|  |  | 50 × 47 (52 × 49) |  | 32 33 |  | 48.0 49.5 |  | 41.569 42.868 |  | 49.7 51.7 |  | 46.7 48.7 |  | 49.78 51.78 |  | 46.92 48.92 |  | + 0.175 + 0.425 |  | 2.558 2.847 |  | 2.75 2.75 |  | 44.141 46.115 |  | 1.87 1.80 |  | 3.00 3.00 |  | 53.138 55.026 |  | 1.55 1.52 |  | 6 6 |  | 24.855 25.175 |  |
|  |  | 55 × 52 (58 × 55) |  | 35 37 |  | 52.5 55.5 |  | 45.466 48.064 |  | 54.7 57.7 |  | 51.7 54.7 |  | 54.79 57.79 |  | 51.91 54.91 |  | + 0.425 + 0.425 |  | 2.847 2.847 |  | 2.75 2.75 |  | 49.119 52.122 |  | 1.79 1.79 |  | 3.00 3.00 |  | 58.038 61.049 |  | 1.53 1.54 |  | 7 7 |  | 29.396 29.535 |  |
|  |  | 60 × 57 (62 × 59) |  | 38 40 |  | 57.0 60.0 |  | 49.363 51.962 |  | 59.7 61.7 |  | 56.7 58.7 |  | 59.79 61.79 |  | 56.91 58.91 |  | + 0.675 + 0.175 |  | 3.136 2.558 |  | 2.75 2.75 |  | 54.181 56.150 |  | 1.75 1.84 |  | 3.00 3.00 |  | 63.047 65.160 |  | 1052 1058 |  | 7 7 |  | 29.855 29.495 |  |

| Table 4 Dimensions for Involute Splines of Module 2 (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia | Measurement Between pine |  |  | Deviation Factor | Pin Dia |  | Measurement Over Pins |  | Deviation Factor |  | Tooth Thickness Over z' Teeth |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth thickness deviation factor 0.866 |
|  | d1× d2 |  | z | d0 |  | db | d3 | d4 | d5 Min | d6 Max | xm |  | lo = so |  | d | Mi |  |  | fi |  | d |  | Ma |  | fa | Z' |  |
|  | 15 × 11 17 × 13 |  | 6 7 | 12 14 |  | 10.392 12.124 | 14.6 16.6 | 10.6 12.6 | 14.68 16.68 | 10.92 12.92 | + 0.4 + 0.4 |  | 3.603 3.603 |  | 3.5 3.5 | 7.629 9.324 |  |  | 2.42 2.19 |  | 5.5 5.0 |  | 22.212 22.695 |  | 1.11 1.13 | 2 2 | 9.121 9.214 |
|  | 18 × 14 20 × 16 |  | 7 8 | 14 16 |  | 12.124 13.856 | 17.6 19.6 | 13.6 15.6 | 17.68 19.68 | 13.92 15.92 | + 0.9 + 0.9 |  | 4.181 4.181 |  | 3.5 3.5 | 10.379 12.736 |  |  | 1.61 1.66 |  | 6.0 6.0 |  | 25.588 28.206 |  | 1.06 1.11 | 2 2 | 9.714 9.807 |
|  | 22 × 18 25 × 21 |  | 9 11 | 18 22 |  | 15.588 19.053 | 21.6 24.6 | 17.6 20.6 | 21.68 24.68 | 17.92 20.92 | + 0.9 + 0.4 |  | 4.181 3.603 |  | 3.5 3.5 | 14.460 17.478 |  |  | 1.64 1.96 |  | 5.5 4.5 |  | 28.790 29.898 |  | 1.13 1.28 | ‒ ‒ | ‒ ‒ |
|  | 28 × 24 30 × 26 |  | 12 14 | 24 28 |  | 20.785 24.249 | 27.6 29.6 | 23.6 25.6 | 27.68 29.69 | 23.92 25.91 | + 0.9 ‒ 0.1 |  | 4.181 3.026 |  | 3.5 3.5 | 20.738 22.484 |  |  | 1.68 2.41 |  | 5.0 4.0 |  | 34.161 34.144 |  | 1.23 1.46 | 3 3 | 15.621 14.807 |
|  | 32 × 28 35 × 31 |  | 14 16 | 28 32 |  | 24.249 27.713 | 31.6 34.6 | 27.6 30.6 | 31.69 34.69 | 27.91 30.91 | + 0.9 + 0.4 |  | 4.181 3.603 |  | 3.5 3.5 | 24.738 27.711 |  |  | 1.69 1.88 |  | 4.5 4.0 |  | 37.016 39.000 |  | 1.30 1.42 | 3 3 | 15.807 15.493 |
|  | 37 × 33 38 × 34 |  | 17 18 | 34 36 |  | 29.445 31.177 | 36.6 37.6 | 32.6 33.6 | 36.69 37.69 | 32.91 33.91 | + 0.4 ‒ 0.1 |  | 3.603 3.026 |  | 3.5 3.5 | 29.571 30.566 |  |  | 1.86 2.15 |  | 4.0 4.0 |  | 40.857 42.181 |  | 1.42 1.50 | 4 3 | 21.028 15.179 |
|  | 40 × 36 42 × 38 |  | 18 20 | 36 40 |  | 31.177 34.641 | 39.6 41.6 | 35.6 37.6 | 39.69 41.69 | 35.91 37.91 | + 0.9 ‒ 0.1 |  | 4.181 3.026 |  | 3.5 3.5 | 32.739 34.589 |  |  | 1.70 2.08 |  | 4.5 4.0 |  | 45.137 46.195 |  | 1.35 1.52 | 4 4 | 21.621 20.807 |
|  | 45 × 41 47 × 43 |  | 21 22 | 42 44 |  | 36.373 38.105 | 44.6 46.6 | 40.6 42.6 | 44.69 46.69 | 40.91 42.91 | + 0.4 + 0.4 |  | 3.603 3.603 |  | 3.5 3.5 | 37.604 39.720 |  |  | 1.84 1.84 |  | 4.0 4.0 |  | 48.938 51.074 |  | 1.46 1.47 | 4 4 | 21.400 21.493 |
|  | 48 × 44 50 × 46 |  | 22 24 | 44 48 |  | 38.105 41.569 | 47.6 49.6 | 43.6 45.6 | 47.69 49.69 | 43.91 45.91 | + 0.9 ‒ 0.1 |  | 4.181 3.026 |  | 3.5 3.5 | 40.740 42.621 |  |  | 1.70 2.00 |  | 4.0 4.0 |  | 51.912 54.218 |  | 1.43 1.54 | 5 4 | 27.435 21.179 |
|  | (52 × 48) 55 × 51 |  | 24 26 | 48 52 |  | 41.569 45.033 | 51.6 54.6 | 47.6 50.6 | 51.69 54.70 | 47.91 50.90 | + 0.9 + 0.4 |  | 4.181 3.603 |  | 3.5 3.5 | 44.740 47.724 |  |  | 1.71 1.82 |  | 4.0 4.0 |  | 55.939 59.109 |  | 1.44 1.50 | 5 5 | 27.621 27.307 |
|  | (58 × 54) 60 × 56 |  | 28 28 | 56 56 |  | 48.497 48.497 | 57.8 59.6 | 53.6 55.6 | 57.70 59.70 | 53.90 55.90 | ‒ 0.1 + 0.9 |  | 3.026 4.181 |  | 3.5 3.5 | 50.642 32.740 |  |  | 1.95 1.71 |  | 4.0 4.0 |  | 62.235 63.984 |  | 1.56 1.47 | 5 6 | 26.933 33.435 |
|  | (62 × 58) 65 × 61 |  | 30 31 | 60 62 |  | 51.962 53.694 | 61.6 64.6 | 54.6 60.6 | 61.70 64.70 | 57.90 60.90 | ‒ 0.1 + 0.4 |  | 3.026 3.603 |  | 3.5 3.5 | 54.650 57.648 |  |  | 1.93 1.80 |  | 4.0 4.0 |  | 66.242 69.058 |  | 1.57 1.53 | 5 6 | 27.179 33.214 |
|  | (68 × 64) 70 × 66 |  | 32 34 | 64 68 |  | 55.426 58.890 | 67.6 69.6 | 63.6 65.6 | 67.70 69.70 | 63.90 65.90 | + 0.9 ‒ 0.1 |  | 4.181 3.026 |  | 3.5 3.5 | 60.740 62.663 |  |  | 1.71 1.90 |  | 4.0 4.0 |  | 72.021 74.253 |  | 1.49 1.59 | 6 6 | 33.807 32.993 |
|  | (72 × 68) 75 × 71 |  | 34 36 | 68 72 |  | 58.890 62.354 | 71.6 74.6 | 67.6 70.6 | 71.70 74.70 | 67.90 70.90 | + 0.9 + 0.4 |  | 4.181 3.603 |  | 3.5 3.5 | 64.740 67.726 |  |  | 1.71 1.79 |  | 4.0 4.0 |  | 76035 79.166 |  | 1.50 1.55 | 7 7 | 39.435 39.121 |
|  | (78 × 74) 80 × 76 |  | 38 38 | 76 76 |  | 65.818 65.818 | 77.6 79.6 | 73.6 75.6 | 77.70 79.70 | 73.90 75.90 | ‒ 0.1 + 0.9 |  | 3.026 4.181 |  | 3.5 3.5 | 70.672 72.740 |  |  | 1.88 1.72 |  | 4.0 4.0 |  | 82.263 84.063 |  | 1.60 1.52 | 7 7 | 38.807 39.807 |
|  | (82 × 78) |  | 40 | 80 |  | 69.282 | 81.6 | 77.6 | 81.70 | 77.90 | ‒ 0.1 |  | 3.026 |  | 3.5 | 74.676 |  |  | 1.87 |  | 4.0 |  | 86.267 |  | 1.61 | 7 | 38.993 |

| Table 5 Dimensions for Involute Splines of Module 2.5 (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. |  |  |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  |  |  | External Spline |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  |  | Measurement Between pine |  |  | Deviation Factor | Pin Dia |  |  | Measurement Over Pins | Deviation Factor | Tooth Thickness Over z' Teeth |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |
|  | d1 × d2 | z | d0 | db | d3 |  | d4 |  | d5 Min |  | d6 Max |  |  | xm | lo = so | d | Mi | fi | d | Ma |  |  | fa |  | Z' |  |  |  |
|  | 20 × 15 22 × 17 | 6 7 | 15.0 17.5 | 12.990 15.155 | 19.5 21.5 |  | 14.5 16.5 |  | 19.58 21.38 |  | 14.92 16.92 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 10.552 12.105 | 1.71 1.85 | 9.0 7.0 | 33.258 30.558 |  |  | 1.03 1.08 |  | 2 2 | 12.026 11.892 |  |  |
|  | 25 × 20 28 × 23 | 8 10 | 20.0 25.0 | 17.321 21.651 | 24.5 27.5 |  | 19.5 22.5 |  | 24.58 27.58 |  | 19.92 22.92 |  |  | + 1.125 + 0.125 | 5.226 4.071 | 4.5 4.25 | 15.552 19.116 | 1.72 2.30 | 7.0 5.0 | 34.113 33.006 |  |  | 1.13 1.37 |  | 2 2 | 12.259 11.491 |  |  |
|  | 30 × 25 32 × 27 | 10 11 | 25.0 27.5 | 21.651 23.816 | 29.5 31.5 |  | 24.5 26.5 |  | 29.58 31.59 |  | 24.92 26.91 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 20.552 22.265 | 1.72 1.81 | 6.5 6.0 | 38.151 38.835 |  |  | 1.19 1.23 |  | 3 3 | 19.293 19.160 |  |  |
|  | 35 × 30 37 × 32 | 12 13 | 30.0 32.5 | 25.981 28.146 | 34.5 36.5 |  | 29.5 31.5 |  | 34.59 36.59 |  | 29.91 31.91 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 25.552 27.308 | 1.72 1.80 | 6.0 5.5 | 42.093 42.764 |  |  | 1.25 1.30 |  | 3 3 | 19.526 19.392 |  |  |
|  | 38 × 33 40 × 35 | 14 14 | 35.0 35.0 | 30.311 30.311 | 37.5 39.5 |  | 32.5 34.5 |  | 37.59 39.53 |  | 32.91 34.91 |  |  | + 0.125 + 1.125 | 4.071 5.226 | 4.5 4.5 | 28.316 30.552 | 2.26 1.72 | 5.0 6.0 | 43.093 47.204 |  |  | 1.43 1.28 |  | 3 3 | 18.759 19.759 |  |  |
|  | 42 × 37 45 × 40 | 15 16 | 37.5 40.0 | 32.476 34.641 | 41.5 44.5 |  | 36.5 39.5 |  | 41.59 44.59 |  | 36.91 39.91 |  |  | + 0.875 + 1.125 | 4.937 5.226 | 4.5 4.5 | 32.340 35.552 | 1.79 1.73 | 5.5 5.5 | 47.881 51.035 |  |  | 1.33 1.33 |  | 3 4 | 19.625 26.793 |  |  |
|  | 47 × 42 48 × 43 | 17 18 | 42.5 45.0 | 36.806 38.971 | 46.5 47.5 |  | 41.5 42.5 |  | 46.59 47.59 |  | 41.91 42.91 |  |  | + 0.875 + 0.125 | 4.937 4.071 | 4.5 4.5 | 37.365 38.387 | 1.78 2.07 | 5.5 5.0 | 52.974 53.156 |  |  | 1.36 1.47 |  | 4 4 | 26.660 26.026 |  |  |
|  | 50 × 45 (52 × 47) | 18 19 | 45.0 47.5 | 38.971 41.136 | 49.5 51.5 |  | 44.5 46.5 |  | 49.59 51.59 |  | 44.91 46.91 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 40.552 42.384 | 1.73 1.78 | 5.5 5.5 | 56.100 58.052 |  |  | 1.36 1.38 |  | 4 4 | 27.026 26.892 |  |  |
|  | 55 × 50 (58 × 53) | 20 22 | 50.0 55.0 | 43.301 74.631 | 54.5 57.5 |  | 49.5 52.5 |  | 54.59 57.60 |  | 49.91 52.90 |  |  | + 1.125 + 0.125 | 5.226 4.071 | 4.5 4.5 | 45.552 48.424 | 1.73 1.99 | 5.5 5.0 | 61.157 63.198 |  |  | 1.38 1.51 |  | 4 4 | 27.259 26.491 |  |  |
|  | 60 × 65 (62 × 57) | 22 23 | 55.0 57.0 | 47.631 49.796 | 59.5 61.5 |  | 54.5 56.5 |  | 59.60 61.60 |  | 54.90 56.90 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 50.552 52.413 | 1.73 1.77 | 5.5 5.0 | 66.206 66.846 |  |  | 1.40 1.45 |  | 5 5 | 34.239 34.160 |  |  |
|  | 65×60 (68×63) | 24 26 | 60.0 65.0 | 51.962 56.292 | 64.5 67.5 |  | 59.5 62.5 |  | 64.60 67.60 |  | 59.90 62.90 |  |  | + 1.125 + 0.125 | 5.226 4.071 | 4.5 4.5 | 55.552 58.448 | 1.73 1.94 | 5.0 5.0 | 69.924 73.229 |  |  | 1.44 1.53 |  | 5 5 | 34.526 33.759 |  |  |
|  | 70×65 (72×67) | 26 27 | 65.0 67.5 | 56.292 58.457 | 69.5 71.5 |  | 64.5 66.5 |  | 69.60 71.60 |  | 64.90 66.90 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 60.552 62.434 | 1.73 1.77 | 5.0 5.0 | 74.954 76.920 |  |  | 1.46 1.48 |  | 5 5 | 34.759 34.625 |  |  |
|  | 75×70 (78×73) | 28 30 | 70.0 75.0 | 60.622 64.952 | 74.5 77.5 |  | 69.5 72.5 |  | 74.60 77.60 |  | 69.90 72.90 |  |  | + 1.125 + 0.125 | 5.226 4.071 | 4.5 4.5 | 65.552 68.464 | 1.73 1.90 | 5.0 5.0 | 79.981 83.253 |  |  | 1.47 1.55 |  | 6 6 | 41.793 41.026 |  |  |
|  | 80×75 (82×77) | 30 31 | 75.0 77.5 | 64.952 67.117 | 79.5 81.5 |  | 74.5 76.5 |  | 79.60 81.60 |  | 74.90 76.90 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 70.552 72.449 | 1.73 1.76 | 5.0 5.0 | 85.004 86.978 |  |  | 1.48 1.50 |  | 6 6 | 42.026 41.892 |  |  |
|  | 85×80 (88×83) | 32 34 | 80.0 85.0 | 69.282 73.612 | 84.5 87.5 |  | 79.5 82.5 |  | 84.60 87.60 |  | 79.90 82.90 |  |  | + 1.125 + 0.125 | 5.226 4.071 | 4.5 4.5 | 75.552 78.476 | 1.73 1.88 | 5.0 5.0 | 90.026 93.273 |  |  | 1.49 1.57 |  | 6 6 | 42.259 41.491 |  |  |
|  | 90×85 (92×87) | 34 35 | 85.0 87.5 | 73.612 75.777 | 89.5 91.5 |  | 84.5 86.5 |  | 89.60 91.60 |  | 84.90 86.90 |  |  | + 1.125 + 0.875 | 5.226 4.937 | 4.5 4.5 | 80.552 82.461 | 1.73 1.76 | 5.0 5.0 | 95.045 97.024 |  |  | 1.50 1.52 |  | 7 7 | 49.293 49.160 |  |  |
|  | 95×90 (98×93) | 36 38 | 90.0 95.0 | 77.942 82.272 | 94.5 97.5 |  | 89.5 92.5 |  | 94.60 97.60 |  | 89.90 92.90 |  |  | + 1.125 + 0.125 | 5.226 4.071 | 4.5 4.5 | 85.552 88.485 | 1.73 1.86 | 5.0 5.0 | 100.063 103.288 |  |  | 1.51 1.58 |  | 7 7 | 49.526 48.759 |  |  |
|  | 100×95 (105×100) | 38 40 | 95.0 100.0 | 82.272 86.603 | 99.5 104.5 |  | 94.5 99.5 |  | 99.60 104.60 |  | 94.90 99.90 |  |  | + 1.125 + 1.125 | 5.226 5.226 | 4.5 4.5 | 90.552 95.552 | 1.73 1.73 | 5.0 5.0 | 105.079 110.094 |  |  | 1.52 1.53 |  | 7 8 | 49.759 56.793 |  |  |

| Table 6 Dimensions for Involute Splines of Module 3 (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. |  |  |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  | Measurement Between pine |  | Deviation Factor |  | Pin Dia |  | Measurement Over Pins |  | Deviation Factor |  | Tooth Thickness Over z' Teeth |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth thickness deviation factor 0.866 |
|  |  | d1 × d2 |  |  |  | z |  | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so | d |  | Mi |  | fi |  | d |  | Ma |  | fa |  | Z' |  |  |
|  |  | 22 × 16 25 × 19 |  |  |  | 6 7 |  | 18 21 | 15.588 18.187 | 21.4 24.4 | 15.4 18.4 | 21.48 24.48 | 15.92 18.92 | + 0.35 + 0.35 | 5.117 5.177 | 5.00 5.00 |  | 11.694 14.235 |  | 2.42 2.18 |  | 7 7 |  | 30.099 32.544 |  | 1.17 1.17 |  | 2 2 |  | 13.431 13.571 |
|  |  | 28 × 22 30 × 24 |  |  |  | 8 8 |  | 24 24 | 20.785 20.785 | 27.4 29.4 | 21.4 23.4 | 27.48 29.48 | 21.92 23.92 | + 0.35 + 1.35 | 5.117 6.271 | 5.25 5.25 |  | 16.835 19.105 |  | 2.56 1.66 |  | 7 8 |  | 36.289 40.013 |  | 1.23 1.14 |  | 2 2 |  | 13.710 14.710 |
|  |  | 32 × 26 35 × 29 |  |  |  | 9 10 |  | 27 30 | 23.383 25.981 | 31.4 34.4 | 25.4 28.4 | 31.49 34.49 | 25.91 28.91 | + 0.85 + 0.85 | 5.694 5.694 | 5.25 5.25 |  | 20.686 24.089 |  | 1.85 1.86 |  | 7 7 |  | 39.577 43.158 |  | 1.21 1.25 |  | 2 2 |  | 14.350 14.490 |
|  |  | 39 × 31 38 × 32 |  |  |  | 11 11 |  | 33 33 | 28.579 28.579 | 36.4 37.4 | 30.4 31.4 | 36.49 37.49 | 30.91 31.91 | + 0.35 + 0.85 | 5.117 5.694 | 5.25 5.25 |  | 25.627 26.762 |  | 2.14 1.83 |  | 6 7 |  | 42.581 45.828 |  | 1.35 1.25 |  | 2 3 |  | 14.129 22.791 |
|  |  | 40 × 34 42 × 36 |  |  |  | 12 12 |  | 36 36 | 31.177 31.177 | 39.4 41.4 | 33.4 35.4 | 39.49 41.49 | 33.91 35.91 | + 0.35 + 1.35 | 5.117 6.271 | 5.25 5.25 |  | 28.964 31.107 |  | 2.10 1.68 |  | 6 7 |  | 45.989 50.023 |  | 1.38 1.26 |  | 3 3 |  | 22.431 23.431 |
|  |  | 45 × 39 47 × 41 |  |  |  | 14 14 |  | 42 42 | 36.373 36.373 | 44.4 46.4 | 38.4 40.4 | 44.49 46.49 | 38.91 40.91 | ‒ 0.15 + 0.85 | 4.539 5.694 | 5.25 5.25 |  | 33.726 36.096 |  | 2.41 1.82 |  | 6 6 |  | 51.216 52.848 |  | 1.46 1.37 |  | 3 3 |  | 22.210 23.210 |
|  |  | 48 × 42 50 × 44 |  |  |  | 14 15 |  | 42 45 | 36.373 38.971 | 47.4 49.4 | 41.4 43.4 | 47.49 49.49 | 41.91 43.91 | + 1.35 + 0.85 | 6.271 5.694 | 5.25 5.25 |  | 37.108 38.855 |  | 1.69 1.80 |  | 7 6 |  | 56.148 55.606 |  | 1.29 1.38 |  | 3 3 |  | 23.710 23.350 |
|  |  | (52 × 46) (55 × 49) |  |  |  | 16 17 |  | 48 51 | 41.569 44.167 | 51.4 54.4 | 45.4 48.4 | 51.49 54.50 | 45.91 48.90 | + 0.35 + 0.35 | 5.117 5.117 | 5.25 5.25 |  | 41.010 43.807 |  | 1.97 1.95 |  | 6 6 |  | 58.088 60.873 |  | 1.44 1.44 |  | 3 3 |  | 22.990 23.129 |
|  |  | (58 × 52) 60 × 54) |  |  |  | 18 18 |  | 54 54 | 46.765 46.765 | 57.4 59.4 | 51.4 53.4 | 57.50 59.50 | 51.90 53.90 | + 0.35 + 1.35 | 5.117 6.271 | 5.25 5.25 |  | 47.024 49.109 |  | 1.94 1.70 |  | 6 7 |  | 64.125 68.343 |  | 1.46 1.34 |  | 4 4 |  | 31.431 32.431 |
|  |  | (62 × 56) 65× 59 |  |  |  | 19 20 |  | 57 60 | 49.363 51.962 | 61.4 64.4 | 55.4 58.4 | 61.50 64.50 | 55.90 58.90 | + 0.85 + 0.85 | 5.694 5.694 | 5.25 5.25 |  | 50.908 54.101 |  | 1.79 1.79 |  | 6 6 |  | 67.767 70.999 |  | 1.43 1.44 |  | 4 4 |  | 32.071 32.210 |
|  |  | (68 × 62) 70 × 64) |  |  |  | 21 22 |  | 63 66 | 54.560 57.158 | 67.4 69.4 | 61.4 63.4 | 67.50 69.50 | 61.90 63.90 | + 0.85 + 0.35 | 5.694 5.117 | 5.25 5.25 |  | 56.928 59.042 |  | 1.78 1.89 |  | 6 6 |  | 73.827 76.183 |  | 1.45 1.49 |  | 4 4 |  | 32.350 31.990 |
|  |  | (72 × 66) 75 × 69) |  |  |  | 22 24 |  | 66 72 | 57.158 62.354 | 71.4 74.4 | 65.4 68.4 | 71.50 74.50 | 65.90 68.90 | + 1.35 ‒ 0.15 | 6.271 4.539 | 5.25 5.25 |  | 61.109 63.932 |  | 1.70 2.00 |  | 6 6 |  | 77.868 81.326 |  | 1.43 1.54 |  | 5 4 |  | 41.152 31.769 |
|  |  | (78 × 72) 80 × 74) |  |  |  | 24 25 |  | 72 75 | 62.354 64.952 | 77.4 79.4 | 71.4 73.4 | 77.50 79.50 | 71.90 73.90 | + 1.35 + 0.85 | 6.271 5.694 | 5.25 5.25 |  | 67.110 68.957 |  | 1.71 1.78 |  | 6 6 |  | 83.909 85.923 |  | 1.44 1.48 |  | 5 5 |  | 41.431 41.071 |
|  |  | (82 × 76) 85 × 79) |  |  |  | 26 27 |  | 78 81 | 67.550 70.148 | 81.4 84.4 | 75.4 78.4 | 81.50 84.50 | 75.90 78.90 | + 0.35 + 0.35 | 5.117 5.117 | 5.25 5.25 |  | 71.054 73.923 |  | 1.86 1.85 |  | 6 6 |  | 88.227 91.092 |  | 1.52 1.52 |  | 5 5 |  | 40.710 40.850 |
|  |  | (88 × 82) 90 × 84) |  |  |  | 28 28 |  | 84 84 | 72.746 72.746 | 87.4 89.4 | 81.4 83.4 | 87.50 89.50 | 81.90 83.90 | + 0.35 + 1.35 | 5.117 6.271 | 5.25 5.25 |  | 77.059 79.110 |  | 1.85 1.71 |  | 6 6 |  | 94.254 95.977 |  | 1.53 1.47 |  | 5 6 |  | 40.990 50.152 |
|  |  | (92 × 86) 95 × 89) |  |  |  | 29 30 |  | 87 90 | 75.344 77.942 | 91.4 94.4 | 85.4 88.4 | 91.50 94.50 | 85.90 88.90 | + 0.85 + 0.85 | 5.694 5.694 | 5.25 5.25 |  | 80.978 84.105 |  | 1.77 1.77 |  | 6 6 |  | 97.995 101.141 |  | 1.49 1.51 |  | 6 6 |  | 49.791 49.931 |
|  |  | (98 × 92) 100 × 94) |  |  |  | 31 32 |  | 93 96 | 80.540 83.138 | 97.4 99.4 | 91.4 93.4 | 97.50 99.50 | 91.90 93.90 | + 0.85 +0 .35 | 5.694 5.117 | 5.25 5.25 |  | 86.987 89.066 |  | 1.77 1.83 |  | 6 6 |  | 104.025 106.275 |  | 1.51 1.55 |  | 6 6 |  | 50.071 49.710 |
|  |  | 105 × 99 110 × 101 |  |  |  | 34 35 |  | 102 105 | 88.335 90.933 | 104.4 109.4 | 98.4 103.4 | 104.51 109.51 | 98.90 103.89 | ‒ 0.15 + 0.85 | 4.539 5.694 | 5.25 5.25 |  | 93.994 99.001 |  | 1.90 1.76 |  | 6 6 |  | 111.380 116.076 |  | 1.59 1.53 |  | 6 7 |  | 49.490 58.791 |
|  |  | 120 × 114 130 × 124 |  |  |  | 38 42 |  | 114 126 | 98.727 109.119 | 119.4 129.4 | 113.4 123.4 | 119.51 129.51 | 113.89 123.89 | + 1.35 + 0.35 | 6.271 5.117 | 5.25 5.25 |  | 109.111 119.078 |  | 1.72 1.81 |  | 6 6 |  | 129.095 136.329 |  | 1.52 1.58 |  | 7 8 |  | 59.710 67.431 |
|  |  | 140 × 134 150 × 144 |  |  |  | 45 48 |  | 135 144 | 116.913 124.708 | 139.4 149.4 | 133.4 143.4 | 139.51 149.51 | 133.89 143.89 | + 0.85 + 1.35 | 5.694 6.271 | 5.25 5.25 |  | 129.026 139.111 |  | 1.76 1.72 |  | 6 6 |  | 146.168 156.172 |  | 1.57 1.55 |  | 8 9 |  | 68.350 77.431 |

| Table 7 Dimensions for Involute Splines of Module 4 (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  |  | Measur‒ ement Between pine |  | Devia‒ tion Factor |  |  |  | Pin Dia |  | Measur‒ement Over Pins |  | Devia‒ tion Factor |  | Tooth Thickness Over z' Teeth |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |
|  | d1 × d2 |  | z | d0 |  | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so |  | d |  |  | Mi |  | fi | d |  |  | Ma |  | fa |  | Z' |  |  |
|  | 32 × 23 35 × 27 |  | 6 7 | 24 28 |  | 20.785 24.249 | 31.2 34.2 | 23.2 26.2 | 31.28 34.29 | 23.92 26.91 | + 1.8 + 1.3 | 8.362 7.784 |  | 7 7 |  |  | 17.470 19.778 |  | 1.64 1.80 | 15 11 |  |  | 54.521 48.316 |  | 1.02 1.09 |  | 2 2 |  | 19.241 18.928 |
|  | 37 × 29 38 × 30 |  | 8 8 | 32 32 |  | 27.713 27.713 | 36.2 37.2 | 28.2 29.2 | 36.29 37.29 | 28.91 29.91 | + 0.3 + 0.8 | 6.630 7.207 |  | 6.75 7 |  |  | 22.935 23.337 |  | 2.42 2.13 | 9 10 |  |  | 47.335 50.447 |  | 1.25 1.19 |  | 2 2 |  | 18.114 18.614 |
|  | 40 × 32 42 × 34 |  | 8 9 | 32 36 |  | 27.713 31.177 | 39.2 41.2 | 31.2 33.2 | 39.29 41.29 | 31.91 33.91 | + 1.8 + 0.8 | 8.362 7.207 |  | 7 7 |  |  | 25.473 26.837 |  | 1.66 2.03 | 12 9 |  |  | 56.413 51.497 |  | 1.11 1.23 |  | 2 2 |  | 19.614 18.800 |
|  | 45 × 37 47 × 39 |  | 10 10 | 40 40 |  | 34.641 34.641 | 44.2 46.2 | 36.2 38.2 | 44.29 46.29 | 36.91 38.91 | + 0.3 + 1.3 | 6.630 7.784 |  | 7 7 |  |  | 30.113 32.472 |  | 2.41 1.81 | 8 10 |  |  | 52.967 59.393 |  | 1.36 1.21 |  | 2 2 |  | 18.486 19.486 |
|  | 48 × 40 50 × 42 |  | 10 11 | 40 44 |  | 34.641 38.105 | 47.2 49.2 | 39.2 41.2 | 47.29 49.29 | 39.91 41.91 | + 1.8 + 0.8 | 8.362 7.207 |  | 7 7 |  |  | 33.475 34.955 |  | 1.67 1.96 | 10 9 |  |  | 60.090 59.979 |  | 1.20 1.28 |  | 3 3 |  | 30.869 30.055 |
|  | (52 × 44) 55 × 47 |  | 11 12 | 44 48 |  | 38.105 41.569 | 51.2 54.2 | 43.2 46.2 | 51.29 54.29 | 43.91 46.91 | + 1.8 + 1.3 | 8.362 7.784 |  | 7 7 |  |  | 37.023 40.474 |  | 1.66 1.79 | 10 9 |  |  | 63.660 65.139 |  | 1.21 1.29 |  | 3 3 |  | 31.055 30.741 |
|  | (58 × 50) 60 × 52 |  | 13 14 | 52 56 |  | 45.033 48.497 | 57.2 59.2 | 49.2 51.2 | 57.30 59.30 | 49.90 51.90 | + 0.8 ‒ 0.2 | 7.207 6.052 |  | 7 7 |  |  | 43.037 44.967 |  | 1.91 2.41 | 8 8 |  |  | 65.470 68.288 |  | 1.37 1.46 |  | 3 3 |  | 30.428 29.614 |
|  | (62 × 54) 65 × 67 |  | 14 15 | 56 60 |  | 48.497 51.962 | 61.2 64.2 | 53.2 56.2 | 61.30 64.30 | 53.90 56.90 | + 0.8 + 0.3 | 7.207 6.630 |  | 7 7 |  |  | 47.411 49.965 |  | 1.91 2.05 | 8 8 |  |  | 69.932 72.791 |  | 1.39 1.43 |  | 3 3 |  | 30.614 30.300 |
|  | (68 × 60) 70 × 62 |  | 16 16 | 64 64 |  | 55.426 55.426 | 67.2 69.2 | 59.2 61.2 | 67.30 69.30 | 59.90 61.90 | ‒ 0.2 + 0.8 | 6.052 7.207 |  | 7 7 |  |  | 53.066 55.421 |  | 2.25 1.88 | 8 8 |  |  | 76.329 78.001 |  | 1.48 1.42 |  | 3 3 |  | 29.986 30.986 |
|  | (72 × 64) 75 × 67 |  | 16 17 | 64 68 |  | 55.426 58.890 | 71.2 74.2 | 63.2 66.2 | 71.30 74.30 | 63.90 66.90 | + 1.8 + 1.3 | 8.362 7.784 |  | 7 7 |  |  | 57.478 60.189 |  | 1.69 1.77 | 9 9 |  |  | 82.163 85.115 |  | 1.33 1.35 |  | 4 4 |  | 42.896 42.555 |
|  | (78 × 70) 80 × 72 |  | 18 18 | 72 72 |  | 62.354 62.354 | 77.2 79.2 | 69.2 71.2 | 77.30 79.30 | 69.90 71.90 | + 0.8 + 1.8 | 7.207 8.362 |  | 7 7 |  |  | 63.429 65.478 |  | 1.86 1.70 | 8 9 |  |  | 86.058 90.273 |  | 1.44 1.35 |  | 4 4 |  | 42.241 43.241 |
|  | (82 × 74) 85 × 77 |  | 19 20 | 76 80 |  | 65.818 69.282 | 81.2 84.2 | 73.2 76.2 | 81.30 84.30 | 73.90 76.90 | + 0.8 + 0.3 | 7.207 6.630 |  | 7 7 |  |  | 67.178 70.341 |  | 1.85 1.95 | 8 8 |  |  | 89.803 93.257 |  | 1.44 1.49 |  | 4 4 |  | 42.428 42.114 |
|  | (88 × 80) 90 × 82 |  | 20 21 | 80 84 |  | 69.282 72.746 | 87.2 89.2 | 79.2 81.2 | 87.30 89.30 | 79.90 81.90 | + 1.8 + 0.8 | 8.362 7.207 |  | 7 7 |  |  | 73.479 75.207 |  | 1.70 1.84 | 9 8 |  |  | 98.368 97.877 |  | 1.37 1.46 |  | 4 4 |  | 43.614 42.800 |
|  | (92 × 84) 95 × 87 |  | 22 22 | 88 88 |  | 76.210 76.210 | 91.2 94.2 | 83.2 86.2 | 91.30 94.30 | 83.90 86.90 | ‒ 0.2 + 1.3 | 6.052 7.784 |  | 7 7 |  |  | 77.215 80.478 |  | 2.04 1.76 | 8 8 |  |  | 100.415 102.993 |  | 1.53 1.45 |  | 4 4 |  | 41.986 43.486 |
|  | (98 × 90) 100 × 92 |  | 23 24 | 92 96 |  | 79.674 83.138 | 97.2 99.2 | 89.2 91.2 | 97.30 99.30 | 89.90 91.90 | + 0.8 ‒ 0.2 | 7.207 6.052 |  | 7 7 |  |  | 83.231 85.243 |  | 1.83 2.00 | 8 8 |  |  | 105.939 108.435 |  | 1.48 1.54 |  | 5 4 |  | 54.055 42.359 |
|  | 105 × 97 110 × 102 |  | 25 26 | 100 104 |  | 86.603 90.067 | 104.2 109.2 | 96.2 101.2 | 104.30 109.31 | 96.90 101.90 | + 0.3 + 0.8 | 6.630 7.207 |  | 7 7 |  |  | 90.181 95.447 |  | 1.89 1.82 | 8 8 |  |  | 113.123 118.217 |  | 1.52 1.50 |  | 5 5 |  | 53.928 54.614 |
|  | 120 × 112 130 × 122 |  | 28 31 | 112 124 |  | 96.995 107.387 | 119.2 129.2 | 111.2 121.2 | 119.31 129.31 | 111.90 121.90 | + 1.8 + 0.8 | 8.362 7.207 |  | 7 7 |  |  | 105.480 115.296 |  | 1.71 1.80 | 8 8 |  |  | 127.969 138.115 |  | 1.47 1.53 |  | 6 6 |  | 66.869 66.428 |
|  | 140 × 132 150 × 142 |  | 34 36 | 136 144 |  | 117.779 124.708 | 139.2 149.2 | 131.2 141.2 | 139.31 149.31 | 131.90 141.90 | ‒ 0.2 + 0.8 | 6.052 7.207 |  | 7 7 |  |  | 125.325 135.458 |  | 1.90 1.79 | 8 8 |  |  | 148.507 158.332 |  | 1.59 1.55 |  | 6 7 |  | 65.986 78.241 |
|  | 160 × 152 170 × 162 |  | 38 41 | 152 164 |  | 131.636 142.028 | 159.2 169.2 | 151.2 161.2 | 159.31 169.31 | 151.90 161.90 | + 1.8 +0.8 | 8.362 7.207 |  | 7 7 |  |  | 145.481 155.342 |  | 1.72 1.78 | 8 8 |  |  | 168.127 178.247 |  | 1.52 1.57 |  | 7 8 |  | 79.614 90.055 |

| Table 8 Dimensions for Involute Splines of Module 5 (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. | Nominal Size |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  | Measur‒ement Between pine |  | Devia‒ tion Factor |  | Pin Dia |  | Measur‒ement Over Pins |  | Devia‒ tion Factor |  | Tooth Thickness Over z' Teeth |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |  |
|  | d1 × d2 | z | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so | d |  | Mi |  | fi |  | d |  | Ma |  | fa |  | Z' |  |  |  |  |
|  | 40 × 30 42 × 32 | 6 7 | 30 35 | 25.981 30.318 | 39 41 | 29 31 | 39.10 41.10 | 29.90 31.90 | + 2.25 + 0.75 | 10.452 8.720 | 9.0 8.5 |  | 21.103 23.559 |  | 1.71 2.18 |  | 18 12 |  | 66.516 55.246 |  | 1.03 1.16 |  | 2 2 |  | 24.052 22.785 |  |  |
|  | 45 × 35 47 × 37 | 7 8 | 35 40 | 30.318 34.641 | 44 46 | 34 36 | 44.10 46.10 | 34.90 36.90 | + 2.25 + 0.75 | 10.452 8.720 | 9.0 9.0 |  | 25.223 27.554 |  | 1.67 2.71 |  | 16 12 |  | 66.185 61.519 |  | 1.04 1.21 |  | 2 2 |  | 24.285 23.017 |  |  |
|  | 48 × 38 50 × 40 | 8 8 | 40 40 | 34.641 34.641 | 47 49 | 37 39 | 47.10 49.10 | 37.90 39.90 | + 1.25 + 2.25 | 9.297 10.452 | 9.0 9.0 |  | 28.922 31.103 |  | 2.13 1.72 |  | 12 14 |  | 62.214 68.226 |  | 1.20 1.13 |  | 2 2 |  | 23.517 24.517 |  |  |
|  | (52 × 42) 55 × 45 | 9 9 | 45 45 | 38.971 38.971 | 51 54 | 41 44 | 51.10 54.10 | 41.90 44.90 | + 0.75 + 2.25 | 8.720 10.452 | 9.0 9.0 |  | 32.028 35.418 |  | 2.41 1.69 |  | 11 14 |  | 63.404 72.552 |  | 1.25 1.13 |  | 2 2 |  | 23.250 24.750 |  |  |
|  | (58 × 48) 60 × 50 | 10 10 | 50 50 | 43.301 43.301 | 57 59 | 47 49 | 57.10 59.10 | 47.90 49.90 | + 1.25 + 2.25 | 9.297 10.452 | 9.0 9.0 |  | 38.968 41.103 |  | 2.01 1.72 |  | 12 14 |  | 72.502 78.660 |  | 1.24 1.17 |  | 2 3 |  | 23.983 38.586 |  |  |
|  | (62 × 52) 65 × 55 | 11 11 | 55 55 | 47.631 47.631 | 61 64 | 51 54 | 61.11 64.11 | 51.89 54.89 | + 0.75 + 2.25 | 8.720 10.452 | 9.0 9.0 |  | 42.254 45.542 |  | 2.19 1.70 |  | 11 12 |  | 73.752 78.376 |  | 1.30 1.22 |  | 2 3 |  | 23.716 38.819 |  |  |
|  | (68 × 58) 70 × 60 | 12 12 | 60 60 | 51.962 51.962 | 67 69 | 57 59 | 67.11 69.11 | 57.89 59.89 | + 1.25 + 2.25 | 9.297 10.452 | 9.0 9.0 |  | 48.996 51.103 |  | 1.95 1.72 |  | 11 12 |  | 80.236 84.186 |  | 1.31 1.25 |  | 3 3 |  | 38.052 39.052 |  |  |
|  | (72 × 62) 75 × 65 | 13 14 | 65 70 | 56.292 60.622 | 71 74 | 61 64 | 71.11 74.11 | 61.89 64.89 | + 0.75 ‒ 0.25 | 8.720 7.565 | 9.0 9.0 |  | 52.398 55.235 |  | 2.08 2.62 |  | 10 10 |  | 81.440 85.360 |  | 1.38 1.46 |  | 3 3 |  | 37.785 37.017 |  |  |
|  | (78 × 68) 80 × 70 | 14 14 | 70 70 | 60.622 60.622 | 77 79 | 67 69 | 77.11 79.11 | 67.89 69.89 | + 1.25 + 2.25 | 9.297 10.452 | 9.0 9.0 |  | 59.014 61.103 |  | 1.91 1.72 |  | 11 12 |  | 90.386 94.408 |  | 1.34 1.28 |  | 3 3 |  | 38.517 39.517 |  |  |
|  | (82 × 72) 85 × 75 | 15 16 | 75 80 | 64.952 69.282 | 81 84 | 71 74 | 81.11 84.11 | 71.89 74.89 | + 0.75 ‒ 0.25 | 8.720 7.565 | 9.0 9.0 |  | 62.489 65.416 |  | 2.01 2.37 |  | 10 10 |  | 91.603 95.411 |  | 1.41 1.48 |  | 3 3 |  | 38.250 37.483 |  |  |
|  | (88 × 78) 90 × 80 | 16 16 | 80 80 | 69.282 69.282 | 87 89 | 77 79 | 87.11 89.11 | 77.89 79.89 | + 1.25 + 2.25 | 9.297 10.452 | 9.0 9.0 |  | 69.027 71.103 |  | 1.88 1.73 |  | 10 11 |  | 97.909 102.069 |  | 1.41 1.33 |  | 3 4 |  | 38.983 53.586 |  |  |
|  | (92 × 82) 95 × 85 | 17 18 | 85 90 | 73.612 77.942 | 91 94 | 81 84 | 91.11 94.11 | 81.89 84.89 | + 0.75 ‒ 0.25 | 8.720 7.565 | 9.0 9.0 |  | 72.574 75.532 |  | 1.96 2.24 |  | 10 10 |  | 101.731 105.453 |  | 1.43 1.50 |  | 3 3 |  | 38.716 37.948 |  |  |
|  | (98 × 88) 100 × 90 | 18 18 | 90 90 | 77.942 77.942 | 97 99 | 87 89 | 97.11 99.11 | 87.89 89.89 | + 1.25 + 2.25 | 9.297 10.452 | 9.0 9.0 |  | 79.036 81.103 |  | 1.86 1.73 |  | 10 11 |  | 107.987 112.210 |  | 1.43 1.36 |  | 4 4 |  | 53.052 54.052 |  |  |
|  | 105 × 95 110 × 100 | 20 21 | 100 105 | 86.603 90.933 | 104 109 | 94 99 | 104.11 109.12 | 94.89 99.89 | ‒ 0.25 ‒ 0.25 | 7.565 7.565 | 9.0 9.0 |  | 85.613 90.366 |  | 2.15 2.11 |  | 10 10 |  | 115.488 120.195 |  | 1.52 1.52 |  | 4 4 |  | 52.017 52.250 |  |  |
|  | 120 × 110 130 × 120 | 22 24 | 110 120 | 95.263 103.923 | 119 129 | 109 119 | 119.12 129.12 | 109.88 119.88 | + 2.25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 101.104 111.104 |  | 1.73 1.73 |  | 10 10 |  | 129.781 139.848 |  | 1.43 1.44 |  | 5 5 |  | 68.586 69.052 |  |  |
|  | 140 × 130 150 × 140 | 26 28 | 130 140 | 112.583 121.244 | 139 149 | 129 139 | 139.12 149.12 | 129.88 139.88 | + 2.25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 121.104 131.104 |  | 1.73 1.73 |  | 10 10 |  | 149.908 159.961 |  | 1.46 1.47 |  | 5 6 |  | 69.517 83.586 |  |  |
|  | 160 × 150 170 × 160 | 30 32 | 150 160 | 129.904 138.564 | 159 169 | 149 159 | 159.12 169.12 | 149.88 159.88 | + 2 .25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 141.104 151.104 |  | 1.73 1.73 |  | 10 10 |  | 170.009 180.052 |  | 1.48 1.49 |  | 6 6 |  | 84.052 84.517 |  |  |
|  | 180 × 170 190 × 180 | 34 36 | 170 180 | 147.224 155.885 | 179 189 | 169 179 | 179.12 189.12 | 169.88 179.88 | + 2.25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 161.104 171.104 |  | 1.73 1.73 |  | 10 10 |  | 190.091 200.126 |  | 1.50 1.51 |  | 7 7 |  | 98.586 99.052 |  |  |
|  | 200 × 190 210 × 200 | 38 40 | 190 200 | 164.545 173.205 | 199 209 | 189 199 | 199.12 209.12 | 189.88 199.88 | + 2.25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 181.104 191.104 |  | 1.73 1.73 |  | 10 10 |  | 210.158 220.188 |  | 1.52 1.53 |  | 7 8 |  | 99.517 113.586 |  |  |
|  | 220 × 210 240 × 230 | 42 46 | 210 230 | 181.865 199.186 | 219 239 | 209 229 | 219.14 239.14 | 209.86 229.86 | + 2.25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 201.104 221.104 |  | 1.73 1.73 |  | 10 10 |  | 230.216 250.264 |  | 1.54 1.55 |  | 8 9 |  | 114.052 128.586 |  |  |
|  | 250 × 240 260 × 250 | 48 50 | 240 250 | 207.846 216.506 | 249 259 | 239 249 | 249.14 258.14 | 239.86 249.86 | + 2.25 + 2.25 | 10.452 10.452 | 9.0 9.0 |  | 231.104 241.104 |  | 1.73 1.73 |  | 10 10 |  | 260.286 270.307 |  | 1.55 1.56 |  | 9 9 |  | 129.052 129.517 |  |  |
|  | 280 × 270 | 54 | 270 | 233.827 | 279 | 269 | 279.14 | 269.86 | + 2.25 | 10.452 | 9.0 |  | 261.104 |  | 1.73 |  | 10 |  | 290.344 |  | 1.57 |  | 10 |  | 144.052 |  |  |

| Table 9 Dimensions for Involute Splines of Module 6 (Clauses 6.2 to 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. | Nominal Size |  |  |  |  |  |  |  |  |  | Internal Spline |  |  | External Spline |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Pin Dia | Measurement Between pine | Deviation Factor | Pin Dia | Measurement Over Pins | Deviation Factor | Tooth Thickness Over z' Teeth |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |
|  |  |  | d1 × d2 |  | z | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so | d |  | Mi |  | fi |  | d |  | Ma |  | fa |  | Z' |  |  |
|  |  |  | 48 × 36 50 × 38 |  | 6 7 | 36 42 | 31.177 36.373 | 46.8 48.8 | 34.8 36.8 | 46.90 48.90 | 35.90 37.90 | + 2.7 + 0.7 | 12.542 10.233 | 10.5 10.5 |  | 26.204 28.471 |  | 1.64 2.18 |  | 22 14 |  | 80.692 65.089 |  | 1.02 1.17 |  | 2 2 |  | 28.862 27.141 |
|  |  |  | (52 × 40) 55 × 43 |  | 7 8 | 42 48 | 36.373 41.569 | 50.8 53.8 | 38.8 41.8 | 50.90 53.90 | 39.90 42.90 | + 1.7 + 0.2 | 11.388 9.653 | 10.5 10.5 |  | 29.136 34.169 |  | 1.89 2.56 |  | 16 12 |  | 71.023 66.901 |  | 1.10 1.33 |  | 2 2 |  | 28.414 26.921 |
|  |  |  | (58 × 46) 60 × 48 |  | 8 8 | 48 48 | 41.569 41.569 | 56.8 58.8 | 44.8 46.8 | 56.90 58.90 | 45.90 47.90 | + 1.7 + 2.7 | 11.388 12.542 | 10.5 10.5 |  | 36.165 38.209 |  | 1.91 1.66 |  | 16 18 |  | 78.701 84.619 |  | 1.16 1.11 |  | 2 2 |  | 28.421 29.421 |
|  |  |  | (62 × 50) 65 × 53 |  | 9 9 | 54 54 | 46.765 46.765 | 60.8 63.8 | 48.8 51.8 | 60.91 63.91 | 49.89 52.89 | + 0.7 + 2.2 | 10.233 11.967 | 10.5 10.5 |  | 39.007 42.406 |  | 2.32 1.73 |  | 14 16 |  | 77.745 84.551 |  | 1.23 1.15 |  | 2 2 |  | 27.700 29.200 |
|  |  |  | (68 × 56) 70 × 58 |  | 10 10 | 60 60 | 51.962 51.962 | 66.8 68.8 | 54.8 56.8 | 66.91 68.91 | 55.89 57.89 | + 0.7 + 1.7 | 10.233 11.388 | 10.5 10.5 |  | 45.840 48.179 |  | 2.24 1.86 |  | 14 14 |  | 84.862 86.315 |  | 1.27 1.25 |  | 2 2 |  | 27.979 28.979 |
|  |  |  | (72 × 60) 75 × 63 |  | 10 11 | 60 66 | 51.962 57.158 | 70.8 73.8 | 58.8 61.8 | 70.91 73.91 | 59.89 62.89 | + 2.7 + 1.2 | 12.542 10.810 | 10.5 10.5 |  | 50.212 52.433 |  | 1.67 1.96 |  | 16 14 |  | 92.508 90.930 |  | 1.18 1.27 |  | 3 3 |  | 46.304 45.083 |
|  |  |  | (78 × 66) 80 × 68 |  | 12 12 | 72 72 | 62.354 62.354 | 76.8 78.8 | 64.8 66.8 | 76.91 78.91 | 65.89 67.89 | ‒ 0.3 + 0.7 | 9.078 10.233 | 10.5 10.5 |  | 55.200 57.928 |  | 2.75 2.10 |  | 12 12 |  | 90.355 91.978 |  | 1.43 1.38 |  | 2 3 |  | 27.538 44.862 |
|  |  |  | (82 × 70) 85 × 73 |  | 12 13 | 72 78 | 62.354 67.550 | 80.8 83.8 | 68.8 71.8 | 80.91 83.91 | 69.89 72.89 | + 1.7 + 0.2 | 11.388 9.653 | 10.5 10.5 |  | 60.187 62.185 |  | 1.84 2.23 |  | 14 12 |  | 98.580 96.601 |  | 1.28 1.41 |  | 3 3 |  | 45.862 44.641 |
|  |  |  | (88 × 76) 90 × 78 |  | 13 14 | 78 84 | 67.550 72.746 | 86.8 88.8 | 74.8 76.8 | 86.91 88.91 | 75.89 77.89 | + 1.7 ‒ 0.3 | 11.388 9.078 | 10.5 10.5 |  | 65.631 67.451 |  | 1.82 2.41 |  | 14 12 |  | 104.032 102.432 |  | 1.29 1.46 |  | 3 3 |  | 46.141 44.421 |
|  |  |  | (92 × 80) 95 × 83 |  | 14 14 | 84 84 | 72.746 72.746 | 90.8 93.8 | 78.8 81.8 | 90.91 93.91 | 79.89 82.89 | + 0.7 + 2.2 | 10.233 11.967 | 10.5 10.5 |  | 69.982 73.233 |  | 2.02 1.75 |  | 12 14 |  | 104.088 111.550 |  | 1.41 1.30 |  | 3 3 |  | 45.421 46.921 |
|  |  |  | (98 × 86) 100 × 88 |  | 15 15 | 90 90 | 77.942 77.942 | 96.8 98.8 | 84.8 86.8 | 96.91 98.91 | 85.89 87.89 | + 0.7 + 1.7 | 10.233 11.388 | 10.5 10.5 |  | 75.529 77.709 |  | 1.99 1.80 |  | 12 12 |  | 109.597 111.211 |  | 1.42 1.38 |  | 3 3 |  | 45.700 46.700 |
|  |  |  | 105 × 93 110 × 98 |  | 16 17 | 96 102 | 83.138 88.335 | 103.8 108.8 | 91.8 96.8 | 103.91 108.92 | 92.89 97.88 | + 1.2 + 0.7 | 10.810 10.233 | 10.5 10.5 |  | 83.132 87.614 |  | 1.88 1.95 |  | 12 12 |  | 117.001 121.745 |  | 1.42 1.44 |  | 3 3 |  | 46.479 46.259 |
|  |  |  | 120 × 108 130 × 118 |  | 18 20 | 108 120 | 93.531 103.923 | 118.8 128.8 | 106.8 116.8 | 118.92 128.92 | 107.88 117.88 | + 2.7 + 1.7 | 12.542 11.388 | 10.5 10.5 |  | 98.217 108.230 |  | 1.70 1.79 |  | 14 12 |  | 136.686 141.998 |  | 1.34 1.44 |  | 4 4 |  | 64.862 64.421 |
|  |  |  | 140 × 128 150 × 138 |  | 22 24 | 132 144 | 114.315 124.708 | 138.8 148.8 | 126.8 136.8 | 138.92 148.92 | 127.88 137.88 | + 0.7 ‒ 0.3 | 10.233 9.078 | 10.5 10.5 |  | 118.085 127.864 |  | 1.89 2.00 |  | 12 12 |  | 152.367 162.653 |  | 1.49 1.54 |  | 4 4 |  | 63.979 63.538 |
|  |  |  | 160 × 148 170 × 158 |  | 25 27 | 150 162 | 129.904 140.296 | 158.8 168.8 | 146.8 156.8 | 158.92 168.92 | 147.88 157.88 | + 1 .7 + 0.7 | 11.388 10.233 | 10.5 10.5 |  | 137.914 147.845 |  | 1.78 1.85 |  | 12 12 |  | 171.846 182.184 |  | 1.48 1.52 |  | 5 5 |  | 82.141 81.700 |
|  |  |  | 180 × 168 190 × 178 |  | 28 30 | 168 180 | 145.492 155.885 | 178.8 188.8 | 166.8 176.8 | 178.92 188.92 | 167.88 177.88 | + 2.7 + 1.7 | 12.542 11.388 | 10.5 10.5 |  | 158.220 168.210 |  | 1.71 1.77 |  | 12 12 |  | 191.953 202.282 |  | 1.47 1.51 |  | 6 6 |  | 100.304 99.862 |
|  |  |  | 200 × 188 210 × 198 |  | 32 34 | 192 204 | 106.277 176.669 | 198.8 208.8 | 186.8 196.8 | 198.92 208.94 | 187.88 197.86 | + 0.7 ‒ 0.3 | 10.233 9.078 | 10.5 10.5 |  | 178.133 187.988 |  | 1.83 1.90 |  | 12 12 |  | 212.550 222.760 |  | 1.55 1.59 |  | 6 6 |  | 99.421 98.979 |
|  |  |  | 220 × 208 240 ×228 |  | 35 38 | 210 228 | 181.865 197.454 | 218.8 238.8 | 206.8 226.8 | 218.94 238.94 | 207.86 227.86 | + 1.7 + 2.7 | 11.388 12.542 | 10.5 10.5 |  | 198.002 218.221 |  | 1.76 1.72 |  | 12 12 |  | 232.152 252.190 |  | 1.53 1.52 |  | 7 7 |  | 117.583 119.421 |
|  |  |  | 250 × 238 260 × 248 |  | 40 42 | 240 252 | 207.840 218.238 | 248.8 258.8 | 236.8 246.8 | 218.94 258.94 | 237.86 247.86 | + 1.7 + 0.7 | 11.388 10.233 | 10.5 10.5 |  | 228.214 238.156 |  | 1.76 1.81 |  | 12 12 |  | 262.447 272.658 |  | 1.55 1.58 |  | 7 8 |  | 118.979 134.862 |
|  |  |  | 280 × 268 |  | 45 | 270 | 233.827 | 278.8 | 266.8 | 278.94 | 267.86 | + 1.7 | 11.388 | 10.5 |  | 258.052 |  | 1.76 |  | 12 |  | 292.335 |  | 1.57 |  | 8 |  | 136.700 |

| Table 10 Dimensions for Involute Splines of Module 8 (Clauses 6.2 6.2 6.10) All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl NO. | Nominal Size |  |  |  |  |  |  |  |  |  | Internal Spline |  |  | External Spline |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  | Measure‒ ment Between Pine | Deviati‒on Factor | Pin Dia | Measurement Over Pins | Deviation Factor | Tooth Thickness Over z' Teeth |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |
|  |  |  | d1 × d2 | z | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so | d |  | Mi |  | fi | d | Ma | fa | Z' |  |
|  |  |  | 60 × 44 65 × 49 | 6 7 | 48 56 | 41.569 48.497 | 58.4 63.4 | 42.4 47.4 | 58.50 63.51 | 43.90 48.89 | + 1.6 + 0.1 | 14.414 12.686 | 14 13 |  | 30.516 36.925 |  | 2.42 2.63 | 22 16 | 88.848 79.195 | 1.11 1.27 | 2 2 | 36.483 35.355 |
|  |  |  | 70 × 54 75 × 59 | 7 8 | 56 64 | 48.497 55.426 | 68.4 73.4 | 52.4 57.4 | 68.51 73.51 | 53.89 58.89 | + 2.6 + 1.1 | 15.569 13.841 | 14 14 |  | 39.557 45.371 |  | 1.80 2.42 | 22 18 | 996.632 95.390 | 1.09 1.24 | 2 2 | 37.855 36.728 |
|  |  |  | 80 × 64 85 × 69 | 8 9 | 64 72 | 55.426 62.354 | 78.4 83.4 | 62.4 67.4 | 78.51 83.51 | 63.89 68.89 | + 3.6 + 2.1 | 16.723 14.996 | 14 14 |  | 50.945 54.803 |  | 1.66 1.89 | 22 20 | 108.241 108.481 | 1.13 1.18 | 2 2 | 39.228 38.100 |
|  |  |  | 90 × 74 95 × 79 | 10 10 | 80 80 | 69.282 69.282 | 88.4 93.4 | 72.4 77.4 | 88.51 93.51 | 73.89 78.89 | + 0.6 + 3.1 | 13.259 16.151 | 14 14 |  | 60.226 65.966 |  | 2.41 1.73 | 16 20 | 105.933 119.486 | 1.36 1.21 | 2 3 | 36.927 61.238 |
|  |  |  | 100 × 84 105 × 89 | 11 12 | 88 96 | 76.210 83.138 | 98.4 103.4 | 82.4 87.4 | 98.51 103.51 | 83.89 88.89 | + 1.6 + 0.1 | 14.414 12.686 | 14 14 |  | 69.911 75.080 |  | 1.96 2.41 | 18 16 | 119.594 121.294 | 1.28 1.41 | 2 2 | 38.345 37.217 |
|  |  |  | 110 × 94 120 × 104 | 12 14 | 96 112 | 83.138 96.995 | 108.4 118.4 | 92.4 102.4 | 108.51 118.51 | 93.89 103.89 | + 2.6 ‒ 0.4 | 15.569 12.104 | 14 14 |  | 80.948 89.935 |  | 1.76 2.41 | 18 16 | 130.279 136.575 | 1.29 1.46 | 3 3 | 61.483 59.228 |
|  |  |  | 130 × 114 140 × 124 | 15 16 | 120 128 | 103.923 110.851 | 128.4 138.4 | 112.4 122.4 | 128.51 138.52 | 113.89 123.89 | + 0.6 + 1.6 | 13.259 14.414 | 14 14 |  | 99.929 110.843 |  | 2.05 1.88 | 16 16 | 145.582 156.001 | 1.43 1.42 | 3 3 | 60.600 61.972 |
|  |  |  | 150 × 134 160 × 144 | 17 18 | 136 144 | 117.779 124.708 | 148.4 158.4 | 132.4 142.4 | 148.51 158.51 | 133.89 143.89 | + 2.6 + 3.6 | 15.569 16.723 | 14 14 |  | 120.378 130.956 |  | 1.77 1.70 | 16 16 | 170.231 180.546 | 1.35 1.35 | 4 4 | 85.110 86.483 |
|  |  |  | 170 × 154 180 × 164 | 20 21 | 160 168 | 138.564 145.492 | 168.4 178.4 | 152.4 162.4 | 168.51 178.51 | 153.89 163.89 | + 0.6 + 1.6 | 13.259 14.414 | 14 14 |  | 140.681 150.414 |  | 1.95 1.84 | 16 16 | 186.514 195.753 | 1.49 1.46 | 4 4 | 84.228 85.600 |
|  |  |  | 190 × 174 200 × 184 | 22 24 | 176 192 | 152.420 166.277 | 188.4 198.4 | 172.4 182.4 | 188.51 198.51 | 173.89 183.89 | + 2.6 ‒ 0.4 | 15.569 12.104 | 14 14 |  | 160.957 170.486 |  | 1.76 2.00 | 16 16 | 205.987 216.870 | 1.45 1.54 | 4 4 | 86.972 84.717 |
|  |  |  | 210 × 194 220 × 204 | 25 26 | 200 208 | 173.205 180.133 | 208.4 218.4 | 192.4 202.4 | 208.52 218.54 | 193.89 203.86 | + 0.6 + 1.6 | 13.259 14.414 | 14 14 |  | 180.363 190.894 |  | 1.89 1.82 | 16 16 | 226.246 236.435 | 1.52 1.50 | 5 5 | 107.855 109.228 |
|  |  |  | 240 × 224 250 × 234 | 28 30 | 224 240 | 193.990 207.846 | 238.4 248.4 | 222.4 232.4 | 238.54 248.54 | 223.86 233.86 | + 3.6 + 0.6 | 16.723 13.259 | 14 14 |  | 210.960 220.788 |  | 1.71 1.86 | 16 16 | 255.938 266.768 | 1.47 1.55 | 6 6 | 133.738 131.483 |
|  |  |  | 260 × 244 280 × 264 | 31 34 | 248 272 | 214.774 235.559 | 258.4 278.4 | 242.4 262.4 | 258.54 278.54 | 243.86 263.86 | + 1.6 ‒ 0.4 | 14.414 12.104 | 14 14 |  | 230.592 250.651 |  | 1.80 1.90 | 16 16 | 276.231 297.014 | 1.53 1.59 | 6 6 | 132.855 131.973 |
|  |  |  | 300 × 284 320 × 304 | 36 38 | 288 304 | 249.415 263.272 | 298.4 318.4 | 282.4 302.4 | 298.54 318.54 | 283.86 303.86 | + 1.6 + 3.6 | 14.414 16.723 | 14 14 |  | 270.915 290.961 |  | 1.79 1.72 | 16 16 | 316.665 336.253 | 1.55 1.52 | 7 7 | 156.483 159.228 |
|  |  |  | 340 × 324 360 × 344 | 41 44 | 328 352 | 284.056 304.841 | 338.4 358.4 | 322.4 342.4 | 338.54 358.54 | 323.86 343.86 | + 1.6 ‒ 0.4 | 14.414 12.104 | 14 14 |  | 310.684 330.731 |  | 1.78 1.85 | 16 16 | 356.494 377.099 | 1.57 1.62 | 7 8 | 158.345 179.228 |
|  |  |  | 380 × 364 400 × 384 | 46 48 | 368 384 | 318.697 332.554 | 378.4 398.4 | 362.4 382.4 | 378.54 398.54 | 363.86 383.86 | + 1.6 + 3.6 | 14.414 16.723 | 14 14 |  | 350.972 370.963 |  | 1.78 1.72 | 16 16 | 396.809 416.458 | 1.58 1.55 | 8 9 | 181.973 206.483 |
|  |  |  | 420 × 404 440 × 424 | 51 54 | 408 432 | 353.338 374.123 | 418.4 438.4 | 402.4 422.4 | 418.56 438.56 | 403.84 423.84 | + 1.6 ‒ 0.4 | 14.414 12.104 | 14 14 |  | 390.739 410.779 |  | 1.77 1.83 | 16 16 | 436.662 437.155 | 1.59 163 | 9 9 | 205.600 204.717 |
|  |  |  | 450 × 434 460 × 444 | 55 56 | 440 448 | 381.051 387.979 | 448.4 458.4 | 432.4 442.4 | 448.56 458.56 | 433.84 443.84 | + 0.6 + 1.6 | 13.259 14.414 | 14 14 |  | 420.697 430.934 |  | 1.80 1.77 | 16 16 | 466.855 476.907 | 1.62 1.60 | 10 10 | 227.855 229.228 |
|  |  |  | 480 × 464 500 × 484 | 58 61 | 464 488 | 401.836 422.620 | 478.4 438.4 | 462.4 482.4 | 478.56 498.56 | 463.84 483.84 | + 3.6 + 1.6 | 16.723 14.414 | 14 14 |  | 450.963 470.776 |  | 1.72 1.77 | 16 16 | 496.602 516.779 | 1.58 1.61 | 11 11 | 253.738 252.855 |

|  |  |  | Table 11 Dimensions for Involute Splines of Module 10 (Clauses 6.2 to 6.10) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | All dimensions in millimetres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl No. |  | Nominal Size |  |  |  |  |  |  |  |  |  |  |  | Internal Spline |  |  |  |  |  | External Spline |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pin Dia |  | Measure‒ ment Between pine |  | Deviation Factor |  | Pin Dia |  | Measurement Over Pins |  | Deviation Factor |  | Tooth Thickness Over z' Teeth |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tooth Thickness Deviation Factor 0.866 |  |
|  | d1 × d2 |  |  | z |  | d0 | db | d3 | d4 | d5 Min | d6 Max | xm | lo = so |  | d |  | Mi |  | fi |  | d |  | Ma |  | fa |  | Z' |  |  |
|  | 80 × 60 85 × 65 |  |  | 6 7 |  | 60 70 | 51.962 60.622 | 78 83 | 58 63 | 78.11 83.11 | 59.89 64.89 | + 4.5 + 2.0 | 20.904 18.017 |  | 18 18 |  | 42.206 44.776 |  | 1.71 2.50 |  | 35 25 |  | 130.845 113.476 |  | 1.03 1.13 |  | 2 2 |  | 48.104 46.069 |
|  | 90 × 70 95 × 75 |  |  | 7 8 |  | 70 80 | 60.622 69.282 | 88 93 | 68 73 | 88.11 93.11 | 69.89 74.89 | + 4.5 + 2.0 | 20.904 18.071 |  | 18 18 |  | 50.446 56.557 |  | 1.67 2.34 |  | 30 25 |  | 127.942 126.117 |  | 1.06 1.19 |  | 2 2 |  | 48.569 46.535 |
|  | 100 × 80 105 × 85 |  |  | 8 9 |  | 80 90 | 69.282 77.942 | 98 103 | 78 83 | 93.11 103.11 | 79.89 84.89 | + 4.5 + 2.0 | 20.904 18.017 |  | 18 18 |  | 62.206 65.378 |  | 1.72 218 |  | 28 25 |  | 136.452 134.743 |  | 1.13 1.19 |  | 2 2 |  | 49.035 47.000 |
|  | 110 × 90 120 × 100 |  |  | 9 10 |  | 90 100 | 77.942 86.603 | 108 118 | 88 98 | 108.11 118.11 | 89.89 99.89 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 70.836 82.207 |  | 1.69 1.72 |  | 28 25 |  | 145.104 150.226 |  | 1.13 1.20 |  | 2 3 |  | 49.500 77.173 |
|  | 130 × 110 140 × 120 |  |  | 12 12 |  | 120 120 | 103.923 103.923 | 128 138 | 108 118 | 128.12 138.12 | 109.88 119.88 | ‒ 0.5 + 4.5 | 15.131 20.904 |  | 16 18 |  | 97.638 102.207 |  | 2.14 1.72 |  | 20 25 |  | 150.592 170.806 |  | 1.43 1.23 |  | 2 3 |  | 45.897 78.104 |
|  | 150 × 130 160 × 140 |  |  | 14 14 |  | 140 140 | 121.244 121.244 | 148 158 | 128 138 | 148.12 158.12 | 129.88 139.88 | ‒ 0.5 + 4.5 | 15.131 20.904 |  | 18 18 |  | 110.471 122.207 |  | 2.62 1.72 |  | 20 25 |  | 170.719 191.284 |  | 1.46 1.27 |  | 3 3 |  | 74.035 79.035 |
|  | 170 × 150 180 × 160 |  |  | 16 16 |  | 160 160 | 138.564 138.564 | 168 178 | 148 158 | 168.12 178.12 | 149.88 159.88 | ‒ 0.5 + 4.5 | 15.131 20.904 |  | 18 18 |  | 130.832 142.207 |  | 2.37 1.73 |  | 20 22 |  | 190.821 204.139 |  | 1.48 1.33 |  | 3 4 |  | 74.966 107.173 |
|  | 190 × 170 200 × 180 |  |  | 18 18 |  | 180 180 | 155.885 155.885 | 188 198 | 168 178 | 188.12 198.12 | 169.88 179.88 | ‒ 0.5 + 4.5 | 15.313 20.904 |  | 18 18 |  | 151.063 162.207 |  | 2.24 1.73 |  | 20 22 |  | 210.906 224.401 |  | 1.50 1.36 |  | 3 4 |  | 75.897 108.104 |
|  | 210 × 190 220 × 230 |  |  | 20 20 |  | 200 200 | 173.205 173.205 | 208 218 | 188 198 | 208.12 218.12 | 189.88 199.88 | ‒ 0.5 + 4.5 | 15.131 20.904 |  | 18 18 |  | 171.225 182.207 |  | 2.15 1.73 |  | 20 22 |  | 230.976 244.627 |  | 1.52 1.38 |  | 4 4 |  | 104.035 109.035 |
|  | 260 × 240 280 × 260 |  |  | 22 24 |  | 220 240 | 190.526 207.846 | 238 248 | 218 228 | 238.14 248.14 | 219.89 229.86 | + 4.5 ‒ 0.5 | 20.904 15.131 |  | 18 18 |  | 202.207 211.440 |  | 1.73 2.04 |  | 22 20 |  | 264.823 271.088 |  | 1.40 1.54 |  | 5 4 |  | 137.173 105.897 |
|  | 260 × 240 280 × 260 |  |  | 24 26 |  | 240 260 | 207.846 225.167 | 258 278 | 238 258 | 258.14 278.14 | 239.86 259.86 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 222.207 242.207 |  | 1.73 1.73 |  | 20 20 |  | 279.696 299.816 |  | 1.44 1.46 |  | 5 5 |  | 138.104 139.035 |
|  | 300 × 280 320 × 300 |  |  | 28 30 |  | 280 300 | 242.487 259.808 | 298 318 | 278 298 | 298.14 318.14 | 279.86 299.86 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 262.207 282.207 |  | 1.73 1.73 |  | 20 20 |  | 319.922 340.017 |  | 1.47 1.48 |  | 6 6 |  | 167.173 168.104 |
|  | 340 × 320 360 × 340 |  |  | 32 34 |  | 320 340 | 277.128 294.449 | 338 358 | 318 338 | 338.14 358.14 | 319.86 339.86 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 302.207 322.207 |  | 1.73 1.73 |  | 20 20 |  | 360.103 380.181 |  | 1.49 1.50 |  | 6 7 |  | 169.035 197.173 |
|  | 380 × 360 400 × 380 |  |  | 36 38 |  | 360 380 | 311.769 329.090 | 378 398 | 358 378 | 378.14 398.14 | 356.86 379.86 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 342.207 362.207 |  | 1.73 1.73 |  | 20 20 |  | 400.252 420.317 |  | 1.51 1.52 |  | 7 7 |  | 198.104 199.035 |
|  | 420 × 400 440 × 420 |  |  | 40 42 |  | 400 420 | 346.410 363.731 | 418 438 | 398 418 | 418.14 438.14 | 399.86 419.84 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 382.207 402.207 |  | 1.73 1.73 |  | 20 20 |  | 440.376 460.431 |  | 1.53 1.54 |  | 8 8 |  | 227.173 228.104 |
|  | 450 × 430 460 × 440 |  |  | 44 44 |  | 440 440 | 381.051 381.051 | 448 458 | 428 438 | 448.16 458.1 6 | 429.84 439.84 | ‒ 0.5 + 4.5 | 15.131 20.904 |  | 18 18 |  | 411.838 422.207 |  | 1.87 1.73 |  | 20 20 |  | 417.374 480.482 |  | 1.62 1.54 |  | 8 8 |  | 224.035 229.035 |
|  | 480 × 460 500 × 480 |  |  | 46 48 |  | 460 480 | 398.372 415.692 | 478 498 | 458 478 | 478.16 498.16 | 459.84 479.84 | + 4.5 + 4.5 | 20.904 20.904 |  | 18 18 |  | 442.207 462.207 |  | 1.73 1.73 |  | 20 20 |  | 500.529 520.572 |  | 1.55 1.55 |  | 9 9 |  | 257.173 258.104 |

| Table 12 Tolerance for Spline Tooth Thickness and Space Width (Clauses 6 and 6.11.3) Values in Micrometres |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl No. |  | Internal Spline Deviations of Space Width Upper ES Lower EI |  |  |  |  |  |  |  |  |  | External Spline Deviation of Tooth Thickness Upper ES Lower EI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | E |  | F |  | G |  | H |  | m |  | k |  | j |  | h |  | g |  | f |  | e |  | d |  | c |  | b |  | A |  |
|  |  | + 45 + 27 |  | + 36 + 18 |  | + 27 + 9 |  | + 18 0 |  | + 27 + 9 |  | + 18 0 |  | + 9 ‒ 9 |  | 0 ‒ 18 |  | ‒ 9 ‒ 27 |  | ‒ 18 ‒ 36 |  | ‒ 27 ‒ 45 |  | ‒ 36 ‒ 54 |  | ‒ 45 ‒ 63 |  | ‒ 54 ‒ 72 |  | ‒ 63 ‒ 81 |  |
|  |  | + 50 + 30 |  | + 40 + 40 |  | + 30 + 10 |  | + 20 0 |  | + 30 + 10 |  | + 20 0 |  | + 10 ‒ 10 |  | 0 ‒ 20 |  | ‒ 10 ‒ 30 |  | ‒ 20 ‒ 40 |  | ‒ 30 ‒ 50 |  | ‒ 40 ‒ 60 |  | ‒ 50 ‒ 70 |  | ‒ 60 ‒ 80 |  | ‒ 70 ‒ 90 |  |
|  |  | + 55 + 33 |  | + 44 + 22 |  | + 33 + 11 |  | + 22 0 |  | + 33 + 11 |  | + 22 0 |  | + 11 ‒ 11 |  | 0 ‒ 22 |  | ‒ 11 ‒ 33 |  | ‒ 22 ‒ 44 |  | ‒ 33 ‒ 55 |  | ‒ 44 ‒ 66 |  | ‒ 55 ‒ 77 |  | ‒ 66 ‒ 88 |  | ‒ 77 ‒ 99 |  |
|  |  | + 62 + 37 |  | + 50 + 25 |  | + 37 + 12 |  | + 25 0 |  | + 37 + 12 |  | + 25 0 |  | + 12 ‒ 13 |  | 0 ‒ 25 |  | ‒ 12 ‒ 37 |  | ‒ 25 ‒ 50 |  | ‒ 37 ‒ 62 |  | ‒ 50 ‒ 75 |  | ‒ 62 ‒ 87 |  | ‒ 75 ‒ 100 |  | ‒ 87 ‒ 112 |  |
|  |  | + 70 + 42 |  | + 56 + 28 |  | + 42 + 14 |  | + 28 0 |  | + 42 + 14 |  | + 28 0 |  | + 14 ‒ 14 |  | 0 ‒ 28 |  | ‒ 14 ‒ 42 |  | ‒ 28 ‒ 56 |  | ‒ 42 ‒ 70 |  | ‒ 56 ‒ 84 |  | ‒ 70 ‒ 98 |  | ‒ 84 ‒ 112 |  | ‒ 98 ‒ 126 |  |
|  |  | + 80 + 48 |  | + 64 + 32 |  | + 48 + 16 |  | + 32 0 |  | + 48 + 16 |  | + 32 0 |  | + 16 ‒ 16 |  | 0 ‒ 32 |  | ‒ 16 ‒ 48 |  | ‒ 32 ‒ 64 |  | ‒ 48 ‒ 80 |  | ‒ 64 ‒ 96 |  | ‒ 80 ‒ 112 |  | ‒ 96 ‒ 128 |  | ‒ 112 ‒ 144 |  |
|  |  | + 90 + 54 |  | + 72 + 36 |  | + 54 + 18 |  | + 36 0 |  | + 54 + 18 |  | + 36 0 |  | + 18 ‒ 18 |  | 0 ‒ 36 |  | ‒ 18 ‒ 54 |  | ‒ 36 ‒ 72 |  | ‒ 54 ‒ 90 |  | ‒ 72 ‒ 108 |  | ‒ 90 ‒ 126 |  | ‒ 108 ‒ 144 |  | ‒ 126 ‒ 162 |  |
|  |  | + 100 + 60 |  | + 80 + 40 |  | + 60 + 20 |  | + 40 0 |  | + 60 + 20 |  | + 40 0 |  | + 20 ‒ 20 |  | 0 ‒ 40 |  | ‒ 20 ‒ 60 |  | ‒ 40 ‒ 80 |  | ‒ 60 ‒ 100 |  | ‒ 80 ‒ 120 |  | ‒ 100 ‒ 140 |  | ‒ 120 ‒ 160 |  | ‒ 140 ‒ 180 |  |
|  |  | + 112 + 67 |  | + 90 + 45 |  | + 67 + 22 |  | + 45 0 |  | + 67 + 22 |  | + 45 0 |  | + 22 ‒ 23 |  | 0 ‒ 45 |  | ‒ 22 ‒ 67 |  | ‒ 45 ‒ 90 |  | ‒ 67 ‒ 112 |  | ‒ 90 ‒ 135 |  | ‒ 112 ‒ 157 |  | ‒ 135 ‒ 180 |  | ‒ 157 ‒ 202 |  |
|  |  | + 125 + 75 |  | + 100 + 50 |  | + 75 + 25 |  | + 50 0 |  | + 75 + 25 |  | + 50 0 |  | + 25 ‒ 25 |  | 0 ‒ 50 |  | ‒ 25 ‒ 75 |  | ‒ 50 ‒ 100 |  | ‒ 75 ‒ 125 |  | ‒ 100 ‒ 150 |  | ‒ 125 ‒ 175 |  | ‒ 150 ‒ 200 |  | ‒ 175 ‒ 225 |  |
|  |  | +140 + 84 |  | + 112 + 56 |  | +84 + 28 |  | + 56 0 |  | + 84 + 28 |  | + 56 0 |  | + 28 ‒ 28 |  | 0 ‒ 56 |  | ‒ 28 ‒ 84 |  | ‒ 56 ‒ 112 |  | ‒ 84 ‒ 140 |  | ‒ 112 ‒ 168 |  | ‒ 140 ‒ 196 |  | ‒ 168 ‒ 224 |  | ‒ 196 ‒ 252 |  |
|  |  | + 158 + 95 |  | + 126 + 63 |  | + 95 + 32 |  | + 63 0 |  | + 95 + 32 |  | + 63 0 |  | + 32 ‒ 31 |  | 0 ‒ 63 |  | ‒ 32 ‒ 95 |  | ‒ 63 ‒ 126 |  | ‒ 95 ‒ 158 |  | ‒ 126 ‒ 189 |  | ‒ 158 ‒ 221 |  | ‒ 189 ‒ 252 |  | ‒ 221 ‒ 284 |  |
|  |  | + 178 + 107 |  | + 142 + 71 |  | + 07 + 36 |  | + 71 0 |  | + 107 ‒ 36 |  | + 71 0 |  | + 36 ‒ 35 |  | 0 ‒ 71 |  | ‒ 36 ‒ 107 |  | ‒ 71 ‒ 142 |  | ‒ 107 ‒ 178 |  | ‒ 142 ‒ 213 |  | ‒ 178 ‒ 249 |  | ‒ 213 ‒ 284 |  | ‒ 249 ‒ 320 |  |
|  |  | + 200 + 120 |  | + 160 + 80 |  | + 20 + 40 |  | + 80 0 |  | + 120 + 40 |  | + 80 0 |  | + 40 ‒ 40 |  | 0 ‒ 80 |  | ‒ 40 ‒ 120 |  | ‒ 80 ‒ 160 |  | ‒ 120 ‒ 200 |  | ‒ 160 ‒ 240 |  | ‒ 200 ‒ 280 |  | ‒ 240 ‒ 320 |  | ‒ 280 ‒ 360 |  |
|  |  | + 225 + 135 |  | + 180 + 90 |  | + 35 + 45 |  | + 90 0 |  | + 135 + 45 |  | + 90 0 |  | + 45 ‒ 45 |  | 0 ‒ 90 |  | ‒ 45 ‒ 135 |  | ‒ 90 ‒ 180 |  | ‒ 135 ‒ 225 |  | ‒ 180 ‒ 270 |  | ‒ 225 ‒ 315 |  | ‒ 270 ‒ 360 |  | ‒ 315 ‒ 405 |  |
|  |  | + 250 + 150 |  | + 200 + 100 |  | + 50 + 50 |  | + 100 0 |  | + 150 + 50 |  | + 100 0 |  | + 50 ‒ 50 |  | 0 ‒ 100 |  | ‒ 50 ‒ 150 |  | ‒ 100 ‒ 200 |  | ‒ 150 ‒ 250 |  | ‒ 200 ‒ 300 |  | ‒ 250 ‒ 350 |  | ‒ 300 ‒ 400 |  | ‒ 350 ‒ 450 |  |
|  |  | + 275 + 165 |  | + 200 + 110 |  | + 65 + 55 |  | + 110 0 |  | + 165 + 55 |  | + 110 0 |  | + 55 ‒ 55 |  | 0 ‒ 110 |  | ‒ 55 ‒ 165 |  | ‒ 110 ‒ 220 |  | ‒ 165 ‒ 275 |  | ‒ 220 ‒ 330 |  | ‒ 275 ‒ 385 |  | ‒ 330 ‒ 440 |  | ‒ 385 ‒ 495 |  |

Table 13 Example of Effective and Actual Dimensions, Tolerances for Different Fits

(Clause 6.11.4)

| Sl No. |  | Spline Type |  | Tolerances Zone |  |  |  | Fit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Effective |  | Actual |  |  |  |
|  |  | Internal Spline |  | H |  | E |  |  |  |
|  |  |  |  | m |  | h |  |  |  |
|  |  |  |  | k |  | g |  | Press fits |  |
|  |  |  |  | j |  | f |  |  |  |
|  |  | External Spline |  | h |  | e |  | Locating fit |  |
|  |  |  |  | g |  | d |  |  |  |
|  |  |  |  | f |  | c |  | Sliding fits |  |
|  |  |  |  | e |  | b |  |  |  |

ANNEX A

(Clause 6.11.3)

INSTRUCTIONS FOR USING TABLE 12

A‒l GENERAL

A‒l.1 This annex covers the method for reading the tolerance values on space width and tooth thickness of four qualities of involute splines, represented by the quality lines ql,q2,q3,q4, etc. The modules up to

10 have been divided into three ranges, represented by module lines m1m2, m3m4 and m5m6. The pitch circle diameters up to 800 mm have been divided into 6 ranges, represented by the lines p1, p2, p3, p4, etc.

A‒2 PROCEDURE FOR USING THE TABLE

A‒2.1 The procedure has been explained with a particular example of external spline of module 2, pitch circle diameter 120 mm, quality 8 and tolerance m.

A‒2.2 The module line for the spline of module 2 is m3m4. The pitch circle diameter line for 120 mm is p7p8. The pitch the module line m3m4 and pitch circle diameter line p7 p8 meet at a point ‘a’, and from the point ‘a’, a spline line leads down to the quality line q3qa, corresponding to quality 8, meeting at point ‘b’. The tolerance value is read under the tolerance symbol m against the point ‘b’. The tolerance value for the tooth thickness of the spline is + 60 mm and + 20 mm.

ANNEX B

(Foreword)

COMMITTEE COMPOSITION

Transmission Devices Sectional Committee, PGD 33

| Organization | Representative(s) |
| --- | --- |
| Maruti Suzuki India Limited, Gurugram | Shri Vikas Chopra (Chairperson) |
| Elecon Engineering Company Limited, Gujarat | Shri Vrajlal Senjaliya Shri Vilas B. Kalyan (Alternate) Shri Prayasvin B. Patel (Alternate II) |
| Forech India Limited, Sonipat | Shri I. K. Bahl Shri Timir Bhattacharyya (Alternate) |
| Honda Motorcycle and Scooter India Private Limited, Gurugram | Shri Arpan Shukla |
| Indian Institute of Technology, Indore | Shri Neelesh Kumar Jain Dr P. P. Chattaraj (Alternate I) Prof I. A. Palani (Alternate II) |
| Kirloskar Pneumatic Company Limited, Pune | Shri Kannan Loganathan |
| LG Balakrishnan and Bros Limited, Coimbatore | Shri Prabakaran P. Shri Subbiah P. (Alternate I) Shri Pannerselvan M. (Alternate II) |
| Lohia Corp Limited, Kanpur | Shri Prakash Kumar Shri Mahendran M. (Alternate I) Shri Dev Kanth (Alternate II) Shri Amit Jaiswal (Alternate III) |
| Mahindra and Mahindra Limited, Mumbai | Shri Jibin K. Paul |
| Maruti Suzuki India Limited, Gurugram | Shri Amit Talwar Shri Gururaj Ravi (Alternate) |
| Research Designs and Standards Organization (RDSO), Lucknow | Shri Vjay Kumar Goel Shri C. K. Bhatt (Alternate I) Shri S. Panwar (Alternate II) |
| Shanthi Gears Limited, Coimbatore | Shri Muthumadhavan V. |
| Suzlon Energy Limited, New Delhi | Shri Mohamed Naseeruddin |
| TIDC India Limited, Chennai | Shri B. Srinivas Dr K. Krishna Kumar |
| VE Commercial Vehicles Limited, Dewas | Shri Arvind Manohar Shri Mahendran Mohan (Alternate I) Shri Saptharishi Iyer (Alternate II) |
| In Personal Capacity (20/2, At Rabindra Nagar Near Sikshan Bhaban, West Midnapore, West Benagal 721101) | Shri Ashish Kumar Khanra |
| In Personal Capacity (D‒1, Kailashpuri Complex, Kusum Vihar. Phase‒2, Koylanagar, Dhanbad ) | Shri K. K. S. Sinha |
| In Personal Capacity (20/2, At Rabindra Nagar Near Sikshan Bhaban, West Midnapore, West Benagal 721101) | Shri Ashish Kumar Khanra |
| BIS Directorate General | Shri R. R. Singh, Scientist ‘F’/SENIOR DIRECTOR and Head (Production and General Engineering) [Representing Director General (Ex‒officio)] |

Member Secretary

Shri Vichitra Vir Singh

Scientist ‘D’/Joint Director

(Production and General Engineering), BIS