भारतीय मानक

Indian Standard

IS 15449 (Part 2) : 2024

घरेलू प्रयोजनों के लिए जिग-जैग सिलाई मशीन/हैड

भाग 2 परिशुद्धता अपेक्षाएँ

( पहला पुनरीक्षण )

Household Zig-Zag Sewing Machine/Head

Part 2 Accuracy Requirements

( First Revision )

ICS 61.080

BIS 2024

भारतीय मानक ब्यूरो

BUREAU OF INDIAN STANDARDS

मानक भवन, 9 बहादुर शाह ज़फर मार्ग, नई दिल्ली - 110002

MANAK BHAVAN, 9 BAHADUR SHAH ZAFAR MARG

NEW DELHI - 110002

www.bis.gov.in     www.standardsbis.in

September 2024                                        Price Group X

Sewing Machines Sectional Committee, MED 29

FOREWORD

This Indian Standard (Part 2) (First Revision) was adopted by the Bureau of Indian Standards, after the draft finalized by the Sewing Machine Sectional Committee had been approved by the Mechanical Engineering Division Council.

This standard was first published in 2004. This standard is being revised to keep pace with the latest technological developments and international practices. Also, in this revision, the standard has been brought into the latest style and format of Indian Standards. The following major modifications have been incorporated in this revision of the standard:

- Title has been changed;

- Scope has been amended to include electronically controlled zig-zag operation;

- Functional dimensions has been updated and another note has been added to cover the rotary hook mechanism (full rotation) machines in 2.1;

- Assembly clearance has been amended to include accuracy requirements for rotary hook mechanism in 2.2;

- Figure 3 in Annex A for needle and shuttle has been updated; and

- Relevant changes have been made in Annex A.

This standard has been formulated to facilitate standardization and with a view to establish quality and accuracy requirements of household zig-zag sewing machine/head, which includes machines with mechanical/ electronically operated zig-zag operations.

In the preparation of this standard, assistance has been derived from IS 7491 : 1989 ‘Sewing machine, household — Accuracy requirements (first revision)’.

The standard on household zig-zag sewing machine, which includes machines with mechanical/electronically operated zig-zag operations, is being brought in four parts, the other parts in the series are:

Part 1  General requirements

Part 3  Sewing requirements

Part 4  Durability requirements

This standard covers all types of zig-zag sewing machine/head, excluding embroidery sewing machines.

The composition of the Committee responsible for the formulation of this standard is given in Annex B.

For the purpose of deciding whether a particular requirement of this standard is complied with, the final value, observed or calculated, expressing the result of a test or analysis, shall be rounded off in accordance with                        IS 2 : 2022 ‘Rules for rounding off numerical values (second revision)’. The number of significant places retained in the rounded off value should be the same as that of the specified value in this standard.

Indian Standard

HOUSEHOLD ZIG-ZAG SEWING MACHINE/HEAD

PART 2 ACCURACY REQUIREMENTS

( First Revision )

1 SCOPE

This standard (Part 2) covers the accuracy requirements for household zig-zag sewing machine/head, which includes machines with mechanical/electronically operated zig-zag operations.

NOTE — This standard covers all types of zig-zag sewing machine/head, excluding embroidery sewing machines.

2 ACCURACY REQUIREMENT

2.1 Functional Dimensions

Clearance and heights at various points on the machine shall be as follows:

| Sl No. | Item | Measuring Condition | Measuring Direction | Ref to Figure | Standard Assembly Dimension mm |
| --- | --- | --- | --- | --- | --- |
| i) | Feed dog | Height of the feed dog teeth above needle/throat plate | Feed dog raised to its highest position | 1 | 1.2, Max |
| ii) | Shuttle and shuttle driver1) | Clearance between heel of shuttle and corresponding tip of shuttle driver | Shuttle heel at bottom most position | 2 | 0.25, Min 0.55, Max |
| iii) | Needle and shuttle for oscillation mechanism Needle and shuttle for rotary hook | Clearance between needle and corresponding face of shuttle Clearance between needle and corresponding face of shuttle | Needle at bottom most position Needle at bottom most position | 3 | 0, Min 0.05, Max 0, Min 0.10 Max |
| iv) | Presser foot | Gap between pressure foot and needle/throat plate | Presser foot in lifted condition | 4 | 5.0, Min |
| v) | Width (zig-zag) | Distance between two extreme needle positions when needle vibration adjustment is at its maximum | Zig-zag width knob or control at its maximum position | 5A 5B | 5.01), Min |
| vi) | Disc follower1) | Clearance between disc follower pin and the highest point of disc no. 1 | Needle bar at its bottom most position | 6 | 0.25, Max |
| NOTES 1 For sewing machines with rotary hook mechanism (full rotation – top loaded or front loaded) in place of oscillating shuttle mechanism requirements under Sl No. (ii) shall not be applicable. 2 Sl No. (vi) shall not be applicable for sewing machine with electronically controlled zig-zag operations. |  |  |  |  |  |

1) Optional requirements depending upon the design specifications of the sewing machine.

2.2 Assembly Clearance

The assembly clearances on the machine at various points shall meet the following requirements:

| Sl No. |  | Item |  | Measuring Condition |  | Measuring Direction |  | Indicator Position |  | Ref to Figure |  | Maximum Assembly Clearance mm |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) |  | (2) |  | (3) |  | (4) |  | (5) |  | (6) |  | (7) |  |
| i) |  | Needle bar |  | Needle bar at lower most position |  | - In the direction of motion - At right angle to the direction of motion |  | - Top of needle bar - Near the bottom of needle bar |  | 7A 7B |  | 0.35 0.20 |  |
| ii) |  | Thread take-up lever |  | Thread take-up lever at top, intermediate and bottom positions |  | - In the direction of motion - At right angle to the direction of motion |  | - Around thread hole - Around thread hole |  | 8A 8B |  | 0.65 0.75 |  |
| iii) |  | Shuttle For rotary hook mechanism |  | - Tip of shuttle pin at top most/bottom most position of needle bar - Difference between two indicator readings when needle bar is at its highest and lowest position With bobbin-case removed, play of bobbin case holder |  | - Along the axis of shuttle pin - Along the axis of shuttle pin - In and out - Up and down |  | - Tip of shuttle pin - Tip of shuttle pin a) On the center pin b) Top of the holder |  | 9 9 |  | 0.20 0.08 0.07 0.16 |  |
| iv) |  | Arm shaft |  | At different arm shaft positions (turn wheel by hand). Axial push/pull to be given: |  | Axial direction |  | Face of rim of wheel: |  | 10 |  | 0.10 |  |
| v) |  | Wheel |  | - Rotate arm shaft to measure wobbling - Rotate arm shaft to measure eccentricity |  | Axial direction Radial direction |  | - Face of rim of wheel - Any point on the periphery of rim of wheel |  | 11A 11B |  | 0.25 0.30 |  |
| vi) |  | Feed section |  | At the highest position of feed dog above needle plate |  | In the direction of motion |  | Front edge of the feed dog |  | 12 |  | 0.35 |  |
| NOTE — The product and figures on measurement methods are for reference purpose only and may vary across different models and manufacturers. |  |  |  |  |  |  |

3 METHOD OF TESTS

The method for carrying out various accuracy tests are given in Annex A.

ANNEX A

(Clause 3)

METHOD OF TESTS

A-1 FEED DOG HEIGHT

For measurement of feed dog height, the arm shaft shall be rotated so as to bring the feed dog to its highest position above the needle plate. The dial indicator shall be so set as to accommodate both the top of the feed dog as well as the needle plate surface within its range. At this position, two readings shall be taken, one at the top of feed dog and the other at the needle plate surface. The difference between these two observations shall indicate the maximum height of the feed dog above the needle plate (see Fig. 1).

Fig. 1 Feed Dog Height

A-2 SHUTTLES AND SHUTTLE DRIVER CLEARANCE

For measurement of the clearance, between shuttle and shuttle driver the machine shall be held in the inclined position. The arm shaft shall be rotated while observing the relative position of the shuttle and its driver till the shuttle heel comes to the bottom most position. At this stage, the measurement for clearance between the heel of the shuttle and the corresponding tip of the shuttle driver shall be made by using a feeler gauge. This clearance indicates the clearance existing between the shuttle and shuttle driver (see Fig. 2).

Fig. 2 Shuttles and Shuttle Driver Clearance

A-3 NEEDLE AND SHUTTLE

For measurement of the clearance, between needle and the corresponding face of shuttle, the machine shall be held in the inclined position. The arm shaft shall be rotated while observing the relative position of the needle and the corresponding face of shuttle, till needle comes to the bottom most position. At this stage, the measurement for clearance between the needle and the corresponding face of the shuttle shall be made by using a feeler gauge. This indicates the clearance existing between the needle and the corresponding face of shuttle (see Fig. 3).

Fig. 3 Full Rotary Needle and Shuttle Clearance

A-4 PRESSER FOOT GAP

For measurement of the presser foot gap, the presser foot shall be lifted to its highest position. The gap between the bottom face of presser foot and the surface of needle plate shall be measured with a suitable gauge. This gap indicates the maximum clearance available between the presser foot and the needle plate of a sewing machine (see Fig. 4).

Fig. 4 Pressure Foot Gap

A-5 ZIG-ZAG WIDTH

A-5.1 For measurement of the zig-zag width or the distance between two needle penetrations (in the needle plate) on the left and right sides respectively, the zig-zag width knob/on the display control panel shall be set at its maximum position. The feed regulator (FR) thumbscrew/display control panel shall be set at zero position or the feed throw out (FTO) knob shall be adjusted to feed drop position (see Fig. 5A).

A-5.2 The arm shaft shall be rotated to take the two penetrations on a piece of paper (instead of cloth), placed between the needle plate and presser foot. The paper shall be taken out from the machine and the distance between the two penetrations measured with the help of a scale. This measurement shall be the width of the zig-zag sewing machine (see Fig. 5B).

Fig. 5A Zig-Zag Width

5 mm

Fig. 5B Zig-Zag Width

A-6 DISC FOLLOWER

For measurement of the clearance, between disc follower pin and the highest point of disc no. 1, rotate the section of panel upwards as shown in Fig. 6. The arm shaft shall be rotated, while observing the relative position of the disc follower pin and the disc no. 1, till the needle bar comes to its bottom most/top most position. At this stage, the measurement for clearance shall be made by using a feeler gauge. This measurement shall indicate the clearance existing between the disc follower pin and the highest point of disc no. 1 (see Fig. 6).

Fig. 6 Disc Follower

A-7 NEEDLE BAR

A-7.1 With the zig-zag width knob/on the display control panel at straight stitch position, move the needle bar to its lowest position and place the dial indicator on the top of the needle bar using an extension piece, if necessary. Apply light hand pressure at the bottom of the needle bar to push it up and note the indicator reading. Similarly, pull the needle bar down with the same pressure and take the second indicator reading. The difference between the two readings shall indicate the assembly clearance for needle bar in the direction of motion (see Fig. 7A).

A-7.2 With the needle bar to its lowest position and zig-zag width knob/on the display control panel at straight stitch position places the indicator near its lower tip by the side of the needle bar. Apply sideward light hand pressure forward and backward. Take one reading of the indicator at each of these two positions. The difference between the two readings shall indicate the assembly clearance for needle bar in the direction of right angle to that of the motion (see Fig. 7B).

Fig. 7A Assembly Clearance for Needle Bar in the

Direction of Motion

Fig. 7B Assembly Clearance for Needle Bar in the

Direction of Right Angle to the Motion

A-8 THREAD TAKE UP LEVER

A-8.1 Bring the thread take up lever at its top/intermediate/bottom position. Place the indicator above the lever around the thread hole. Pull the lever down with very light hand pressure and note the indicator reading. Similarly, push the lever up with the same pressure and observe the second reading. The difference between the two readings will indicate the clearance in the direction of motion for thread take up lever at its given position (see Fig. 8A).

A-8.2 For the clearance in the right angle direction, place the indicator on the sidewall near the thread hole of the take up lever. Apply very light pressure on the tip of the lever forward and backward and take one reading of the indicator at each of these two positions. The difference between the two readings shall indicate the assembly clearance for thread take up lever in the direction of right angle to that of the motion (see Fig. 8B).

Fig. 8A Thread Take Up Lever

A-8.2 For the clearance in the right angle direction, place the indicator on the sidewall near the thread hole of the take up lever. Apply very light pressure on the tip of the lever forward and backward and take one reading of the indicator at each of these two positions. The difference between the two readings shall indicate the assembly clearance for thread take up lever in the direction of right angle to that of the motion (see Fig. 8B).

Fig. 8B Thread Take Up Lever (Right Angle)

A-9 SHUTTLE

Set the indicator on the top of shuttle pin in the axial direction. Hold the pin and apply very light axial pull and push and take one reading of the indicator at each of these two positions. The difference between the two readings indicates the assembly clearance for the shuttle in the axial direction. An indicator attachment may be necessary for this measurement (see Fig. 9).

Fig. 9 Shuttle

A-10 ARM SHAFT

Place the indicator in contact with the side face of the rim of arm shaft at any convenient point. Apply medium hand pressure and pull and push the arm shaft in the axial direction of the arm shaft. Take one reading of the indicator at each of these two positions. The difference between the two readings shall indicate the clearance for the arm shaft in the axial direction (see Fig. 10).

Fig. 10 Arm Shaft Clearance

A-11 WHEEL

A-11.1 For determination of the wobbling of the arm shaft, bring the indicator in contact with the side face of the rim of the wheel. Rotate the wheel by at least one revolution and note the total difference in indicator reading. This reading will indicate the wobbling of the arm shaft in the axial direction (see Fig. 11A).

A-11.2 For determination of the eccentricity of the arm shaft, bring the indicator in contact with the circumference of the wheel. Rotate the arm shaft by at least one revolution and note the total difference in indicator reading. This reading will indicate the eccentricity of the arm shaft in the radial direction (see Fig. 11B).

Fig. 11A Arm Shaft (Wobbling)

A-11.2 For determination of the eccentricity of the arm shaft, bring the indicator in contact with the circumference of the wheel. Rotate the arm shaft by at least one revolution and note the total difference in indicator reading. This reading will indicate the eccentricity of the arm shaft in the radial direction (see Fig. 11B).

Fig. 11B Arm Shaft (Eccentricity)

A-12 FEED DOG SECTION

For measurement of the feed dog section, the arm shaft shall be rotated so as to bring the feed dog to its highest position above the needle plate. The dial indicator shall be set on the front edge of the feed dog. The maximum feed amount shall be measured with dial indicator in the direction of motion (see Fig. 12).

Fig. 12 Feed Dog Section

ANNEX B

(Foreword)

COMMITTEE COMPOSITION

Sewing Machines Sectional Committee, MED 29

| Organization |  | Representative (s) |  |
| --- | --- | --- | --- |
| Research & Development Centre for Bicycle and Sewing Machines, Ludhiana |  | Shri Sanjeev Katoch (Chairperson) Shri Papinder Singh (Alternate I) Shri Vishwas Mehta (Alternate II) Shri Manpreet Singh (Alternate III) |  |
| Brother International (India) Private Limited, Mumbai |  | Shri Mathew Yohannan |  |
| C.R. Auluck & Sons Private Limited, Ludhiana |  | Shri Sunil Auluck Shri Kuljeet Singh (Alternate) |  |
| Directorate General of Quality Assurance, New Delhi |  | Shri R.V. Jain |  |
| G.D. Rupal Industries, Ludhiana |  | Shri Gurmukh Singh |  |
| Gee Tech Hooks, Ludhiana |  | Shri Manjeet Singh |  |
| Geminy Industrial Enterprises Private Limited, Ludhiana |  | Shri Vinay Dua Shri B.C. Pandey (Alternate) |  |
| Ludhiana Sewing Machine Association, Ludhiana |  | Shri Hardeep Singh Shri Rajvinder (Alternate) |  |
| Makhan Sewing Machines, Ludhiana |  | Shri Dalbir Singh Dhiman |  |
| Mechanical Engineering Research and Development Organization (MERADO) , Ludhiana |  | Shri Syed Salman Mojiz Shri Bhagwant Singh Lal (Alternate) |  |
| Narindera and Company, Ludhiana |  | Shri S. Baldev Singh Shri Harinder Jit Singh (Alternate) |  |
| Navrang Manufacturing Corporation, Ludhiana |  | Shri Dinesh Kapila Shri Sudesh Kapila (Alternate) |  |
| Northern India Textile Research Association, Ghaziabad |  | Shri Vikas Sharma Shri Vivek Agarwal (Alternate) |  |
| Novel Sewing Machine Technologies, Pune |  | Shri Bharat Narayendas Parmar Shri Arjun Bharat Parmar (Alternate) |  |
| ORAA International, Ludhiana |  | Shri Ashish Gupta |  |
| Office of Development Commissioner (MSME), New Delhi |  | Shri Suvankar Santra Ms Maitreyee Talapatra (Alternate) |  |
| Ranew Engineering (India) Private Limited, Ludhiana |  | Shri Sanjeev Kumar Jain Shri Abhilash Jain (Alternate) |  |
| Rita Machines India Private Limited, Ludhiana |  | Shri Sunil Kumar Jain Shri Jagdish Chandra Auluck (Alternate) |  |
| Singer India Limited, New Delhi |  | Shri Prashant Aggarwal Shri Atul Kumar Seth (Alternate) |  |
| Swan Mechanical Works, Ludhiana |  | Shri Amarjeet Singh |  |
| United Sewing Machines and Parts Manufacturing Association, Ludhiana |  | Shri Dalbir Singh Dhiman |  |
| Usha International Limited, New Delhi |  | Shri Rup Lal Kangla Shri Pranay Sriwastav (Alternate) |  |
| Uttam Sewing Machine Company (Private) Limited, Jalandhar |  | Shri Jagdeep Rai Shri Manohar Lal (Alternate) |  |
| Virindra Engineering Works, Ludhiana |  | Shri Amarpreet Singh Panesar Shri Swarn Singh (Alternate) |  |
| Voluntary Organisation in Interest of Consumer Education (VOICE), New Delhi |  | Shri M. A. U. Khan |  |
| BIS Directorate General |  | Shri K. Venkateswara Rao, Scientist ‘F’/Senior Director and Head (Mechanical) [Representing Director General (Ex-officio)] |  |

Member Secretary

Shri Shubham Tiwari

Scientist ‘D’/Joint Director

(Mechanical), BIS